BroadNAS System Manufacturing

Version: 1.30

April 14, 2005

- Revision Chart

| Version | Description of Version | Date Completed |
| --- | --- | --- |
| 1.00 | Document created starting from the Manufacturing Test Design document | 10/8/2004 |
| 1.10 | Corrected manufacturing test sample output. Modified Preparing BroadNAS System for Manufacturing section to reflect the changes in the process | 10/14/2004 |
| 1.20 | Added flow chart and information about flashcfetrx.pl script | 12/14/2004 |
| 1.30 | Added detail about using nasload instead of TFTP and information about enhancements added in CFE 1.3.x | 4/14/2005 |

- Contents

1.	Introduction	1

1.1	Scope	1

1.2	References	1

2.	BroadNAS Flash Partitions	2

3.	Preparing for BroadNAS System Manufacturing	3

3.1	Pre-programming Flash	3

3.2	Initial System Configuration	4

3.3	Loading Manufacturing Test Firmware Image	6

3.4	Timing Setup for nasload for Manufacturing	6

4.	Flow Diagram for System Startup	7

5.	Test Setup	8

5.1	Test Harness for Board Level Test	8

5.2	Test Harness for System Test	8

5.2.1	Block Diagram	9

5.2.2	DHCP Server	9

5.2.3	Test Controller	9

5.2.4	Device Under Test	10

5.3	Setting up the Test Controller	10

5.4	Installing and Running Test Scripts	10

5.5	Running Tests Individually	10

5.6	Running Tests Using the Test Harness	10

6.	Manufacturing Tests	13

6.1	Manufacturing Test Flow	13

6.2	Preliminary board tests	13

6.3	System Manufacturing Tests	14

6.3.1	Visual verification of LEDs	14

6.3.2	Manual verification of buttons and switches	14

6.3.3	Identification of internal and external devices	14

6.3.4	Verfication of internal and external device configuration	14

6.3.5	Ethernet data throughput test	15

6.3.6	Hard drive thoughput test	15

6.3.7	Final Steps	15

6.4	Wireless connectivity test (if WLAN supported on BroadNAS)	15

7.	What does Broadcom provide?	16

7.1	For BroadNAS Target	16

7.2	For Host PC	16

8.	Appendix A. Enhancements in CFE for Manufacturing	17

8.1	Enhancements for CFE 1.3.x	17

# Introduction

The BroadNAS system is a combination of hardware and software components.  Depending on the end-product, the hardware components include the system enclosure, BroadNAS board, enclosure management input and output devices such as, buttons, switches, indicator LEDs and fan, and hard disk drives.  The software components include the CFE boot loader, the run-time Linux kernel, user-space root file system, and system configuration NVRAM variables.

Broadcom provides a typical process for manufacturing a BroadNAS system.  Broadcom also provides the tools required for this.  Some of the tools are provided as sources, so that they can be expanded or customized as per the needs of a specific product and its manufacturing process.

## Scope

This document describes in detail the typical manufacturing flow of the BroadNAS system.  The following is described in detail in this document.

- BroadNAS Flash partitions

- Preparing the BroadNAS system for manufacturing. This includes setting up the board specific information such as MAC address, serial number, etc.

- Setting up and running typical manufacturing tests.

- Tools and utilities provided.

This document does not provide any information on using the serial console or on doing any BroadNAS software development work.

## References

- BCM4780P Resource Guide.

- BroadNAS Manufacturing Test Design document.

- NASoC System Manufacturing Test Getting Started Guide.

# BroadNAS Flash Partitions

The BroadNAS Flash is partitioned as shown below.

Low Address (0)

Partition 1 CFE Boot Loader (with 1KB of Embedded NVRAM Variables at offset 0x1000) (256KB)

Partition 2 & 3 Kernel

Root File System (Maximum size of kernel + root file system for a 8MB Flash = 7.5625MB)

Partition 4 Copy of run-time NVRAM Variables (64 KB)

Partition 5 Run-time NVRAM Variables (64 KB)

High Address (0x7FFFFF)

Notes:

- Different size Flash may be used (2MB, 4MB, 16MB, etc.).

- The Embedded NVRAM Variables region contains the hard configuration NVRAM variables with system specific information such as the MAC address, serial number, etc.

- The root file system partition (partition 3) may contain the pivot root file system required to pivot the run-time root file system from a hard disk drive.

- If the entire run-time firmware is on the hard disk drive, the partitions 2 and 3 will not be used.

# Preparing for BroadNAS System Manufacturing

## Pre-programming Flash

- The ODM is responsible for getting an image pre-programmed on the Flash before it gets soldered on the board.  This image should contain at least the CFE boot loader, and optionally, the kernel, and a root file system.  The ODM can provide this image to the Flash manufacturer to pre-program the Flash parts with this image.  The flashpack.sh shell script can be used to generate an image with CFE, kernel, and root file system as a single binary image that can then be programmed in the Flash.

IMPORTANT: For running the manufacturing tests, the kernel and the root file system is required.  Also, the run-time NVRAM region is required to be initialized to some pattern as described below.

- The run-time NVRAM region in this image should be blank or initialized to some pattern, but should not contain any valid data.  If the system needs to go into manufacturing test state automatically, the run-time NVRAM region HAS TO BE initialized to some 32-bit pattern (other than 0xFFFFFFFF).  Also, the goto_mfgstate_pattern NVRAM variable has to be defined as this pattern (for eg. goto_mfgstate_pattern=ab12) in the board-specific CFE image.

Note: Currently, the flashpack.sh scripts allows to pad the unused portion of the Flash (including the run-time NVRAM regions) to be all 0s only.  Hence, the only pattern supported is all 0s.

- The CFE image contains default embedded NVRAM variables with values appropriately set to match the hardware.  The value of the cfe_configuration_state NVRAM variable in the embedded region must be set to not_initalized if the image is not unique for each board/system (i.e. it has a common MAC address).  If the image is unique for the board/system (has a unique MAC address), the value of cfe_configuration_state must be set to initialize.

IMPORTANT: If the Flash part is not pre-programmed with at least the CFE boot loaer, ODM will have to program the image to the on-board Flash via JTAG.  Also, if the Flash is pre-programmed with a common image, the cfe_configuration_state NVRAM variable has to be set to not_initialized.  This will make CFE wait for a board-specific image to be downloaded over the network and programmed in the CFE partition of the Flash.  This is required if the system does not have any serial console.

- When board/system boots with cfe_configuration_state NVRAM variable set to not_initialized, CFE will be in a state waiting or a board-specific CFE image to be downloaded over the network and programmed in the CFE partition of the Flash.

- If the system needs to obtain the IP address via DHCP in this state, the cfe_mfg_lan_proto NVRAM variable should be defined as dhcp_client.  If this NVRAM variable is not defined or defined as something else, the IP address will be set statically.

- Since each board/system requires a unique MAC address (and optionally other information, such as, a serial number), a unique image will have to be created for each Flash device or that information can be modified during production as described in the Intial System Configuration section.

- The MAC address (and any other unique information, such as the serial number) is part of the embedded NVRAM variable region which is at 1 KB region at offset 4 KB within CFE.  If the Flash devices have not been pre-programmed with unique images, this unique information will be needed to be programmed as the very first thing on the production floor.  The process for doing this is described in the Intial System Configuration section.

## Initial System Configuration

- If the Flash is pre-programmed with a common Flash image, make sure that the following NVRAM variables are set.

  - configuration_state NVRAM variable is set to not_initialized

  - The IP address should be set such that the system will be accessible on the production floor network or the cfe_mfg_lan_proto is set to dhcp_client for the system to obtain the IP address via DHCP.

  - If the autodiscovery from client PC is to be used, make sure that the cfe_password_protect_tftp is set to enabled.  Else TFTP will be needed to be used to download the new CFE image.

- Program the board-specific CFE.  This can be done by either of the following two ways.

(1) Manual Process

  - Create a new board/system specific CFE boot loader image as described below.

    - Run the nvserial utility to create a board specific CFE binary image using the NVRAM text file and specifying the last two octets of the MAC address.  For example -

nvserial -i cfe.bin -o cfenas.bin -s 0xEC00  bcm94780nas.txt

This will create the board specific cfenas.bin image with the last two octets of the MAC address as EC00.  The top 4 octets of the MAC address and the rest of the NVRAM variables will be set from the bcm94780nas.txt file.

    - Make sure the cfe_configuration_state NVRAM variable is set to initialized in the NVRAM variable text file.  Also, for running the manufacturing tests, make sure the the NVRAM variable goto_mfgstate_pattern is set to the value of the pattern that the run-time NVRAM region is programmed with.  If DHCP is to be used for manufacturing, cfe_mfg_lan_proto needs to be set to dhcp_client and if nasload is to be used to download the image, cfe_password_protect_tftp needs to be set to enabled.

    - Run the trx utility to add a TRX header to the binary created by the nvserial command in the step above.  For example -

trx –o cfenas.trx –f 2 cfenas.bin

This creates the CFE binary with the TRX header which contains the proper flags to identify this as a CFE image and CRC.  Program this file as the new CFE.

  - Connect the system to be programmed to the network and power it ON.  The system will have the default MAC address and IP address.  Make sure that the system is on the same subnet as the default IP address or is using DHCP protocol.

  - TFTP the cfenas.trx image in binary mode to the IP address of the BroadNAS system.  If using nasload insteade, the following command can be used (please do nasload /? for details about the options).

nasload /a /U /P /f cfe.trx

(2) Automated Perl Script (flashcfetrx.pl)

- Copy the following files to one directory, either, on a Linux PC or on a Windows PC

cfe.bin		-	CFE binary file

flashcfetrx.pl	-	Perl utility to customize and program CFE

bcm94780nas.txt	-	Or some similar text file with NVRAM variables. Make sure the cfe_configuration_state NVRAM variable is set to initialized in the NVRAM variable text file.  Also, for running the manufacturing tests, make sure the the NVRAM variable goto_mfgstate_pattern is set to the value of the pattern that the run-time NVRAM region is programmed with.  If DHCP is to be used for manufacturing, cfe_mfg_lan_proto needs to be set to dhcp_client and if nasload is to be used to download the image, cfe_password_protect_tftp needs to be set to enabled.

nvserial		-	or nvserial.exe for Windows. Utility to embed NVRAM

trx			-	or trx.exe for Windows. Utility to create TRX header

- From the directory where all these files are copied run -

flashcfetrx.pl -board_name mxo94780 -board_number <n>

-board_macaddr <MM:MM:MM:MM:MM:MM> -user_name <uname>

-user_psw <pswd>

where -

n - is the board number

MM:MM:MM:MM:MM:MM - is the mac address

uname - is the user name for the system administrator

pswd - is the password for the system administrator

- Hit Enter at the command prompt.  The program will wait forever for the target.

- Connect the system to be programmed to the network and power it ON.  The system will have the default MAC address and IP address.  Make sure that the system is on the same subnet as the default IP address or is using DHCP protocol.

- When board/system boots with cfe_configuration_state NVRAM variable set to intialized, the CFE boot loader assumes that the unique information has been programmed in the NVRAM variables.

- With the cfe_configuration_state NVRAM variable set to initialized, and no valid data in the run-time NVRAM regions, and if the goto_mfgstate_pattern NVRAM variable is set, CFE shall set the manufacturing_state NVRAM variable to not_tested.  This way on subsequent boots and when the system configuration is restored to factory defaults, the system will not get into manufacturing state.

- With the cfe_configuration_state NVRAM variable set to initialized, and no valid data in the run-time NVRAM regions, CFE shall copy the embedded NVRAM variables there.

- At this point, if the manufacturing_state NVRAM variable is set to not_tested, CFE will wait for the manufacturing test root file system to be loaded over the network to the reserved RAM area.

## Loading Manufacturing Test Firmware Image

After programming the system with board specific CFE with the NVRAM variable manufacturing_state set as not_tested, the system waits for the manufacturing root file system to be loaded over the network into RAM.  Load the manufacturing test root file system (target.cramfs ) image via TFTP or nasload to the IP address of the BroadNAS system.  This image provides support for telnet.  Wait for a few seconds for the system to boot and then run the test scripts from the host PC.  To load the image via nasload, run the following command –

nasload /a /U /P /f target.cramfs

## Timing Setup for nasload for Manufacturing

When the BroadNAS is setup to use nasload there are 2 timing parameters that can be modified.

cfe_announce_timeout

When cfe_password_protect_tftp is enabled, this variable specifies the number of seconds to wait for a download server announcement.  If no value is given (or the value zero is given), CFE waits for 25 seconds.

cfe_auth_timeout

When cfe_password_protect_tftp is enabled, this variable specifies the number of seconds to wait for an authorization request from the download server.  The download server needs to read user input (i.e., username and password) before it can send this message.  If no value is given (or the value zero is given), CFE waits for 300 seconds.  If /U and /P options are used with nasload and no username and password variables are defined, this timing is not required as BroadNAS does not wait for authorization.

# Flow Diagram for System Startup

no

yes

no

yes

initialized

Power-on

Boot

Is not defined or is set to not_initialized

NVRAM variable cfe_configuration_state?

Wait for board-specific CFE to be programmed

Program board specific CFE with configuration_state = initialized

Pattern in run-time NVRAM region  matches  value of  NVRAM variable goto_mfgstate_pattern?

Set manufacturing_state NVRAM variable to not_tested and prepare to load root filesystem in RAM

Is manufacturing_state NVRAM variable set to not_tested?

Load mfg test root file system, run tests and set manufacturing_state to tested if tests pass

Continue with normal boot

# Test Setup

## Test Harness for Board Level Test

The test harness or ICT (In Circuit Tester) used for board level testing is ODM specific.  Hence, the board-level diagnostic test and test process is ODM specific too.

Broadcom does not provide any specific diagnostic test to the ODMs for board level diagnostic tests.

## Test Harness for System Test

The system tests are done at the end of production line.  The basic requirements for the test harness system test is listed below –

- Assembled system under test. This includes LEDs, buttons, switches, and fans.  Battery for real time clock should also be installed.

- If the system has support for WLAN, the WLAN mini-PCI card and antennas shall be installed.

- If hard disk drives are shipped with the system, they should be installed.  If no hard disk drives are shipped, a known good hard disk drive should be attached to all the drive interfaces.

- External known-good USB device (e.g. Flash disk) should be connected to the USB port if present

- Host PC connected via the LAN.

- WLAN AP router and/or WLAN clients to test WLAN connection if BroadNAS system has WLAN capability.

### Block Diagram

DUT

Test

Controller

DHCP

Server

LAN

Scripts and logs

The Manufacturing Test System consists of the following components:

- DHCP server - To assign IP address to the Test Controller and Device Under Test.

- Test Controller - This is the host system which initiates the tests and reports test results.

- Device Under Test - The NASoC system to be tested.

### DHCP Server

A DHCP server is required to assign IP address to the Test Controller and DUT.  The device-under-test is configured to acquire IP address from a DHCP server.

### Test Controller

The Test Controller is a host system which initiates the tests and reports test results.  A set of test scripts are provided to test various hardware on the NASoC system.  These test scripts can be run individually from the command line or they can be run collectively in an automated fashion using the provided test harness.

The features of the Test Controller include:

- Tests can be automated using the provided test harness.

- Host system can be any TCP/IP host that is capable of running perl scripts (e.g. Windows, Linux).

- Test scripts and harness can be configured via ini-style configuration files.

- Logging and statistics are generated.

### Device Under Test

The Device-Under-Test is the NASoC system being tested.  A special Linux root file system for manufacture test is required.  This file system contains utilities to communicate with the Test Controller.  See Loading Manufacturing Test Image section for information on how to load the file system on the DUT.

## Setting up the Test Controller

The Test Controller requires a perl installation and several external perl modules that might not be installed on your host system.  The external perl modules are:

- AppConfig-1.56

- Net-Telnet-3.03

The above perl module can be obtained from http://cpan.org/.

## Installing and Running Test Scripts

The test scripts are written in perl.  They have file extension of .pl.  Each test script has a corresponding configuration file to customize the test.  The configuration files have file extension of .ini.

Because many tests share the same configuration attributes (e.g. IP address of device-under-test, timeout value), a global.ini is provided to specify those common attributes.

Place the test files (*.pl, *.ini, and *.pm) in a directory and cd to that directory.  Review the configuration files and customize them to match your environment.

## Running Tests Individually

The script can be run individually from the command line:

perl <Testname.pl>

Here is an output of the Flash test that has 4 subtests:

C:\> cd \mfgtest

C:\mfgtest> perl flash.pl -target_host 10.19.13.109 -log_dir ./target1

1..4

ok 1 - Test for existance of proc entry /proc/mtd.

ok 2 - Flash is detected.

ok 3 - Verify vendor ID to be Amd/Fujitsu

ok 4 - Verify the size of the flash to be 8388608

## Running Tests Using the Test Harness

A set of tests can be run using the Test Harness.  The list of tests is specified in harness.ini.

Here is an output of the Flash test that was run under the test harness:

C:\mfgtest> perl harness.pl -target_host 10.19.13.109 -log_dir ./target1

#########################################################

#  Start time = Fri Oct  8 01:24:44 2004

#  target_host = 10.19.13.109

#  log_dir = ./target1

#  Test_list = Flash.pl, PCI.pl

#########################################################

Flash....1..4

ok 1 - Test for existance of proc entry /proc/mtd.

ok 2 - Flash is detected.

ok 3 - Verify vendor ID to be Amd/Fujitsu

ok 4 - Verify the size of the flash to be 8388608

ok

PCI......1..50

ok 1 - Test for existance of proc entry /proc/pci.

ok 2 - Look for 12 devices.

ok 3 - device0 with bus_no=0 device_no=0 function=0 is found. isa HASH

ok 4 - device0 has class=0501.

ok 5 - device0 has vendor=14e4.

ok 6 - device0 has device=0800.

ok 7 - device1 with bus_no=0 device_no=1 function=0 is found. isa HASH

ok 8 - device1 has class=0200.

ok 9 - device1 has vendor=14e4.

ok 10 - device1 has device=4713.

ok 11 - device2 with bus_no=0 device_no=2 function=0 is found. isa HASH

ok 12 - device2 has class=0200.

ok 13 - device2 has vendor=14e4.

ok 14 - device2 has device=4713.

ok 15 - device3 with bus_no=0 device_no=3 function=0 is found. isa HASH

ok 16 - device3 has class=0c03.

ok 17 - device3 has vendor=14e4.

ok 18 - device3 has device=4715.

ok 19 - device4 with bus_no=0 device_no=4 function=0 is found. isa HASH

ok 20 - device4 has class=0604.

ok 21 - device4 has vendor=14e4.

ok 22 - device4 has device=0804.

ok 23 - device5 with bus_no=0 device_no=5 function=0 is found. isa HASH

ok 24 - device5 has class=0b30.

ok 25 - device5 has vendor=14e4.

ok 26 - device5 has device=0816.

ok 27 - device6 with bus_no=0 device_no=6 function=0 is found. isa HASH

ok 28 - device6 has class=0703.

ok 29 - device6 has vendor=14e4.

ok 30 - device6 has device=4712.

ok 31 - device7 with bus_no=0 device_no=7 function=0 is found. isa HASH

ok 32 - device7 has class=1000.

ok 33 - device7 has vendor=14e4.

ok 34 - device7 has device=4718.

ok 35 - device8 with bus_no=0 device_no=8 function=0 is found. isa HASH

ok 36 - device8 has class=0500.

ok 37 - device8 has vendor=14e4.

ok 38 - device8 has device=080f.

ok 39 - device9 with bus_no=1 device_no=0 function=0 is found. isa HASH

ok 40 - device9 has class=0600.

ok 41 - device9 has vendor=14e4.

ok 42 - device9 has device=4704.

ok 43 - device10 with bus_no=1 device_no=2 function=0 is found. isa HASH

ok 44 - device10 has class=0101.

ok 45 - device10 has vendor=1166.

ok 46 - device10 has device=0242.

ok 47 - device11 with bus_no=1 device_no=2 function=1 is found. isa HASH

ok 48 - device11 has class=0101.

ok 49 - device11 has vendor=1166.

ok 50 - device11 has device=0242.

ok

All tests successful.

Files=2, Tests=54,  2 wallclock secs

All tests succeeded.

#########################################################

#    End time = Fri Oct  8 01:24:46 2004

# target_host = 10.19.13.109

#     log_dir = ./target1

#########################################################

# Manufacturing Tests

The system manufacturing tests are done at the end of production line after the system has been assembled.

## Manufacturing Test Flow

- For running the manufacturing tests, the NVRAM manufacturing_state has to be set to not_tested.  When the BroadNAS system boots with this setting, it waits for the manufacturing test root file system to be loaded over the network to the RAM.  The image is loaded using TFTP.

- Once the system boots with the manufacturing test root file system, it provides a console over telnet.  The test controller uses this console for running the tests.

- The manufacturing tests shall run when the tester does so from the console prompt, and goes through all the tests that have been selected by the ODM.  The system under test will send the test result strings to a log program running on the host Linux PC.  The test results string specifies, the test run, test result (passed or failed), and some information on why the test failed if it did.  Some of the tests, like performance tests, might not have a pass/fail result.  In that case, the output will specify the performance numbers.

- Some of the tests may require interaction from the tester.  For example, button and switch test shall prompt the tester to press a specific button or switch.  For LED tests, the tester will be prompted to enter whether the LED did what it was supposed to do (On, Off, or blinking).

- Once all the tests are done, the test program will change the manufacturing_state to either tested_ok or test_failed.

- On the next power-cycle the system shall start executing the run-time image programs rather than the manufacturing tests.

## Preliminary board tests

During manufacturing system test process, at power-on, some preliminary board tests shall be run to verify that the board is operational.  These tests are listed below.

- Checksum verification of the run-time Flash image

- Checksum verification of the manufacturing test Flash image

- Partial address and data bus bit tests of SDRAM memory (TBD)

- Access IDE controller over PCI bus and verify vendor and device identification

- Access WLAN mini-PCI card if available and verify vendor and device identification

- Internal and external loop back test for Ethernet (TBD)

## System Manufacturing Tests

### Visual verification of LEDs

Since not all systems will have all the LEDs supported by BroadNAS software, when the ODM configures the tests that they want to run on their system, they also need to configure which LEDs need to be verified.  For example, if the system has only the the “System Ready” LED, the test will be configured to test just that LED.  The test will go through the LEDs configured to be tested and turn it On, blinking, and Off and prompt the tester to verify it.  The test result of each LED shall be logged.  Please refer to LEDs.txt and LEDs.ini files in the manufacturing scripts directory for details about this test.

### Manual verification of buttons and switches

Since not all systems will have all the buttons and switches supported by BroadNAS software, when the ODM configures the tests that they want to run on their system, they also need to configure which buttons and switches need to be verified.  The test will go through the buttons and switches to be tested and prompt the user to press it and verify if the state changed was latched.  The test result of each button and switch shall be logged.  Please refer to Buttons.txt and Buttons.ini files in the manufacturing scripts directory for details about this test.

### Identification of internal and external devices

The test shall query and identify all the devices visible on the PCI bus, SMBUS, and IDE or SATA bus.  All the devices identified shall be logged.  Additional information of all the devices as provided by the manufacturer and the protocol shall also be logged.  For example, vendor ID, and device ID.  Please refere to PCI.txt, PCI.ini, USB.txt, USB.ini, IDE.txt, IDE.ini, SATA.txt, and SATA.ini files in the manufacturing scripts directory for details about how the different devices are detected.  Detection of devices on the SMBUS is not yet available (TBD).

### Verfication of internal and external device configuration

All the internal and external devices are expected to have a certain configuration in the BroadNAS software.  This test shall verify that the devices are configured as expected.  The expected and the actual configuration shall be logged and differences flagged in the log messages.  Since some of the external devices are optional, the ODM shall configure what external devices are expected to be there and their configuration modes.

Listed below are the devices tested and their expected configuration.

Ethernet PHY	-	Auto negotiation mode

USB Controller	-	Host mode

IDE Drives	-	UDMA(5) mode

USB Drives	-	Report bus speed connected

The .txt and .ini files mentioned in the Identification of Internal and External Devices section also provide details on verification of the device configuration.

### Ethernet data throughput test

The test shall transmit packets over LAN to the program running on the host PC which will transmit the packets back.  The test program shall evaluate the time taken and calculate the throughput.  The test statistics (number of packets, packet size and time taken) shall be logged.  The test program shall also verify that the throughput is within reasonable range and log the results.  Please refer to Network.txt and Network.ini files in the manufacturing scripts directory for details about this test.

### Hard drive thoughput test

This test will write raw data to the hard drive(s) attached to the system (ATA, SATA, or USB) and read it back.  The throughput measurement shall be done for write and read.  If the hard drive is pre-formatted and has data on it which cannot be destructed, the ODM shall specify that during configuration of the tests.  In that case, the test shall read data from the file and save it in memory.  A test pattern will then be written to and read from the block(s) from which data was saved.  After the test is done, the original data will be copied back.  The test statistics (data size and time taken) shall be logged.  The test program shall also verify that the throughput is within reasonable ramie and log the results.  Please refer to Disks.txt and Disks.ini files in the manufacturing scripts directory for details about this test.  Currently, this test does not do any data throughput testing.  This will be added in the near future (TBD).

### Final Steps

Once all the tests are done, the manufacturing test phase is over and the test software script needs to change the manufacturing state to tested_ok if the tests have passes or tested_failed if any test fails.  On the next power cycle, the kernel will read the NVRAM variable for manufacturing state and mount the run-time root file system instead of the manufacturing test root file system.  As a final check, few functional tests, including hardware clock, WLAN, etc. will need to be done after the system boots with the shipping run-time root file system.

## Wireless connectivity test (if WLAN supported on BroadNAS)

Note:	This test will actually be done after the manufacturing test phase when the system is in normal operation mode.  All setups and status verification shall be done via the web pages.

BroadNAS will be configured as a server and a client connected to it.  The connection status shall be verified.  Optionally, BroadNAS shall be configured as a client and connected to an AP router.  Also, channel number and rate will be modified and the connection re-tested.

# What does Broadcom provide?

This section provides the list of things that Broadcom provides to the ODM for manufacturing tests.

## For BroadNAS Target

- cfe.bin  - Binary image of the CFE boot loader.

- bcm94780nasdef.txt and bcm94780nas.txt - Sample NVRAM variable texts file for creating default CFE image and board-specific CFE image from the CFE binary file.

- broadnas.trx, broadnas.nas, broadnas_split.nas, broadnas_disk.nas - Binary images of Linux kernel and run-time root file system for different system configurations.

- pivot_root.trx – Binary image of Linux kernel and root file system to load the run-time root file system off a hard disk drive.

- target.cramfs - Binary image of manufacturing test file system (under src/apps/mfgtest_root/mipsel directory).

## For Host PC

- mfgtest directory - Sources for the manufacturing test scripts and documents.

- nvserial and nvserial.exe – Linux and Windows utility for creating board-specific CFE image from the cfe.bin file.

- trx and trx.exe – Linux and Windows utility to add TRX header to the board-specific CFE image so that it can be programmed on the system.

- flashpack.sh – Linux script file to create Flash image that can be pre-programmed on the Flash device.

- flashcfetrx.pl – Perl scipt to customize and program  board-specific CFE.

- nasload – Linux and Windows utility to load an image to the BroadNAS.

# Appendix A. Enhancements in CFE for Manufacturing

## Enhancements for CFE 1.3.x

- If cfe_wait_led_gpio NVRAM variable is defined.  The signal on the GPIO pin number defined by this NVRAM variable will be toggled everytime the system goes through the cycle of waiting for the image to be downloaded.  Each cycle is defined in seconds by the wait_time NVRAM variable.

- Currently, on restore-to-default (by pressing the RESET button and holding it for more than 5 seconds), the system clears the run-time NVRAM regions and restores the values from the embedded configuration information.  In the normal product the system will not wait for anything to be downloaded from the network and CFE will load the firmware from Flash directly.  If cfe_wait_on_restore NVRAM variable is defined, the system will wait for an image to be downloaded over the network and programmed to the Flash.  The wait time is the number of iterations given by the value set for the cfe_wait_on_restore NVRAM variable, where each iteration is wait_time seconds long.  If the value is set to 0, the system will wait for ever.