4780P

Resource

Guide

- Revision Chart

| Version | Description of Version | Date Completed |
| --- | --- | --- |
| 1.0 | First released version. | 6/17/2004 |
| 1.1 | Added information on obtaining and using JTAG controller from EPI for programming on-board Flash. | 7/2/2004 |
| 1.2 | Added caveat about needing to clean linux kernel directory before build. | 7/2/2004 |
| 1.3 | Added chapter 3 on debugging and chapter 4 on loading image in RAM and moved GPIO customization to chapter 5. | 7/25/2004 |
| 1.4 | Added information on setting up console and breaking into CFE | 8/5/2004 |
| 1.5 | Corrected release version information for support for loading image in RAM. Supported in 1.1 and not 1.0 | 8/6/2004 |
| 1.6 | Minor clarifications added to the instructions | 8/18/2004 |
| 1.7 | Added section for default disk and pool actions | 3/29/05 |
| 1.8 | Added section on updating CFE without serial console | 4/14/05 |

- Contents

1.	Introduction to source code	1

1.1	Installing the Tools	1

1.2	Layout of the Source Tree	1

1.3	Recompiling and Changing the Linux Kernel	2

1.4	Creating a New Flashable Image (.trx file)	2

1.5	Adding New Applications to the NASoC Image	3

2.	Bootloader and related utilities	5

2.1	Introduction	5

2.2	CFE boot loader images	5

2.2.1	Creating a board speciic CFE	5

2.3	Accessing CFE from the Console	6

2.3.1	Console Settings	6

2.3.2	Getting CFE Prompt	6

2.4	Programming CFE boot loader on a blank flash	7

2.4.1	Blank Flash NOT soldered on the board	7

2.4.2	Blank Flash or Flash with non-working CFE soldered on the board	7

2.4.2.1	Ordering MAJIC JTAG Controller From EPI	7

2.5	Updating CFE boot loader	8

2.5.1	Updating Existing CFE via command line	8

2.5.2	Updating Existing Firmware via command line	8

2.5.3	Updating Existing CFE via automated script and Serial Console	9

2.5.4	Updating Existing CFE via automated script – No Console	10

2.6	NVRAM variable text file	10

2.6.1	Required NVRAM variables	11

2.6.2	Optional NVRAM variables	13

3.	BroadNAS Software Debugging	14

3.1	EPI Development Kit	14

3.1.1	Setting up EPI Debugging Environment under Windows	14

3.1.2	Using EDB Debugger	15

3.2	BroadNAS Console	15

3.3	Syslog	15

4.	Loading BroadNAS Without Programming Flash	17

4.1	Introduction	17

4.2	Required Setup	17

4.3	Loading CRAMFS Root Filesystem in RAM via TFTP	17

4.4	Loading and booting kernel from RAM	18

5.	Customizing GPIO’s	19

5.1	Introduction	19

5.1.1	BroadNAS System Miscellaneous Inputs and Outputs	19

5.1.1.1	Inputs	19

5.1.1.2	Outputs	19

5.1.2	BCM4780P General Purpose Inputs and Outputs	20

5.2	Hardware Configuration	20

5.2.1	Using CPLD	20

5.2.2	Using 4780P GPIO Pins	21

5.2.3	Selecting the hardware configuration	21

5.3	Software Configuration	21

5.3.1	Broadcom BroadNAS Reference Platform	21

5.3.2	Customizing miscellaneous inputs and outputs for other hardware configuration	22

5.3.2.1	Provide a device driver	22

5.3.2.2	Select and load the device driver	22

5.3.2.3	Entries created by the device driver	23

5.3.2.4	Mapping device I/O to BroadNAS system inputs and outputs	24

6.	Customizing Defaults	25

6.1	Customizing Default Disk & Printer Actions	25

6.2	Extend network nvram variables functionality	25

6.2.1	Adding new functionality for wan_proto	25

6.2.2	Overriding existing functionality for wan_proto	26

6.3	Receiving Event Notifications	27

6.3.1	New Disk Arrival Notification	28

7.	Customizing Temperature sensor and Fan Devices	29

7.1	Sensor Device Driver Interface	29

7.2	Temperature Monitor and Fan Control Daemon	29

7.2.1	Auto Fan Control	29

7.2.2	NVRAM variables	31

8.	Small Flash Support	32

8.1	Configuring an Existing System	32

8.2	NASoC Image Components	33

8.3	NVRAM Variables	33

8.4	NASoC Rocovery	34

9.	Downloading Firmware with NASLOAD	35

9.1	Discovery of NAS on the network	35

9.2	When a Download Program is Required	35

9.3	The NAS Upgrade Protocol	36

9.3.1	Sequence	36

9.3.2	Message Format	37

9.4	NASLOAD	37

9.4.1	Dependencies	37

9.4.2	Command Line	37

9.4.3	Compiling	38

# Introduction to source code

In order to develop for the NASoC platform, a developer needs an x86 RedHat Linux host system (or one capable of running binaries compiled for such) and a NASoC development platform, called BroadNAS. The software comes in two parts – a tarball containing the Mips cross-compilers (named hndtools-3.0.tar.gz for discussion purposes) and a tarball containing the NASoC source code release (named NASoC_rel_1.0.tar.gz in the following examples).

## Installing the Tools

The tools must be installed in /opt/brcm in order to be able to locate libraries and include files. Since the files in the hndtools-3.0.tar.gz tarball are relative to /opt, the following commands will install the tools in the correct location:

mkdir /opt # if it doesn't exist

cd /opt

tar xzf ${PATH_TO_TARBALL}/hndtools-3.0.tar.gz

ln –s /opt/brcm/hndtools-mipsel-linux-3.0 /opt/brcm/hndtools-mipsel-linux

After the tools are installed, add /opt/brcm/hndtools-mipsel-linux/bin to the path. The compiler and utilities (mipsel-linux-gcc et al) reside in the /opt/brcm/hndtools-mipsel-linux/bin directory.

## Layout of the Source Tree

Uncompress NASoC_rel_1.0.tar.gz to get a top-level nasoc directory. Components under this tree include:

+ nasoc

+ doc

+ Release		<- location of quick start guide and release notes

+ tools			<- directory containing trx and other utilities

+ image			<- location in which images are built

+ src

+ linux

+ linux		<- location of Linux kernel

+ apps			<- location of application code

The utilities in the nasoc/tools directory are needed to assemble an image suitable for flashing into the BroadNAS. This directory (nasoc/tools) must also be added to the path.

## Recompiling and Changing the Linux Kernel

The kernel configuration file nasoc/src/linux/linux/.config is derived from nasoc/src/linux/linux/arch/mips/defconfig-bcm94780. To add or change kernel options, perform the following steps:

% cd nasoc/src/linux/linux

% make mrproper

% cp arch/mips/defconfig-bcm94780 .config

<edit .config to make modifications>

% make oldconfig

% make dep

% make zImage modules

This produces a compressed kernel which is located at nasoc/src/linux/linux/arch/mips/brcm-boards/bcm947xx/compressed/vmlinuz. Do not do a “make install” or “make modules-install” from here as these are done automatically from the apps directory to relink an image (see below).

Because the .depend files in the Linux kernel source directory contain references to absolute paths from the Broadcom build environment, you must perform a clean (“make mrproper”) and reconfigure before you can build any of the source code provided. This is likely to be changed in upcoming releases.

Before doing any incremental changes to the kernel   “make” should be issued from the top level source directory. This will resolve all dependencies between different directories.

## Creating a New Flashable Image (.trx file)

To create a new broadnas.trx file from the kernel in nasoc/src/linux/linux and the applications in nasoc/src/apps, you need to execute the following:

% cd nasoc/src/apps

% make all install

This compiles the various applications and installs the appropriate kernel and modules into nasoc/src/apps/mipsel, prepares a root filesystem from nasoc/src/apps/mipsel/target, and binds everything into a .trx file which will be located in nasoc/src/apps/mipsel/broadnas.trx.

If you wish to recompile all of the various applications, you can execute the following:

% cd nasoc/src/apps

% make clean

Do not manually erase objects or executables under this tree as some of the binaries are not reproducible from the released source code tree.

## Adding New Applications to the NASoC Image

To add a new application, a directory under nasoc/src/apps should be created, and the application code installed therein.

A config file (similar to the Linux kernel .config file) is used to control what components are enabled in a particular build. The master copy of this config file resides in nasoc/src/apps/config/defconfig. Edit this file (and the corresponding documentation file nasoc/src/apps/config/Config) to introduce a new CONFIG_APP1 (for example) option to control inclusion/exclusion of this application. Whenever changes to this defconfig are made, you will need to delete nasoc/src/apps/.config and “make .config” in nasoc/src/apps.

Edit the nasoc/src/apps/Makefile to add this directory to the list that is compiled. Usually this corresponds to adding a line such as

obj-$(CONFIG_APP1) += app1_directory_name

in the appropriate location. You may also need to add dependencies if your application depends on other (non-standard) libraries or applications. If you require custom build rules they can also be added – see busybox and busybox-install targets for examples.

Without custom build rules, the default action is to cd into app1_directory_name and run “make” or “make install” as appropriate. When this occurs, CC and various other makefile macros are predefined to appropriate values (see top of nasoc/src/apps/Makefile for complete list). In particular, your application makefile should ensure that the binaries are installed in the directory defined by $(INSTALLDIR).

This INSTALLDIR variable, when your application Makefile is invoked, will point to nasoc/src/apps/mipsel/install/app1_directory_name. Under there, you should install in a path relative to the root directory. For instance, if your application creates a binary file called app1 that you want to be located in /usr/sbin/app1 on the root FS, then you install (and strip) your binary in $(INSTALLDIR)/usr/sbin/app1 which ends up being nasoc/src/apps/mipsel/install/usr/sbin/app1.

Note: do not rely on the “install --strip” option to strip your binary, as this will invoke the host platform’s strip utility. Instead, use the $(STRIP) macro supplied which will invoke the appropriate mipsel-linux-strip. Binaries stripped with the wrong tool may fail to load/run.

Prior to creating the root filesystem image in nasoc/src/apps/mipsel/target, the apps Makefile gathers up and merges the various files from within nasoc/src/apps/mipsel/install. It also creates a special version of the shared library glibc that contains only those modules needed by the binaries that will end up on the root filesystem. This may pose a problem if you intend to compile and run applications without recreating a .trx image every time. There are two ways to solve this problem: you can link your binaries using static libraries (ie use the –static flag to gcc during the link step), or you can create a file named nasoc/src/apps/lib/.custom_binaries, and list the full Linux host paths to any additional executables and shared libraries that you intend to run on the NASoC. Put one file per line. These additional binaries will be searched for glibc functions in the final library optimization step during the creation of a .trx file.

# Bootloader and related utilities

## Introduction

BroadNAS uses CFE (Common Firmware Environment) as the bootloader.  CFE is programmed at the CPU’s boot vector and is the first thing that gets executed at power-on.  It then loads the run-time firmware image from Flash.  There are certain NVRAM variables that are shared between the CFE and the run-time firmware image.  Also, most of the basic hardware setup is done in CFE and used by the run-time firmware image.  This document describes the process to create, program and update CFE image, and NVRAM variables needed for basic functionality.

## CFE boot loader images

There are two CFE images provided as part of the release:

cfe.bin is the binary that gets built from the sources for a specific project .  For NASoC, it gets built under nasoc/src/cfe/build/broadcom/bcm947xx.

cfenas.bin is the combination of the cfe.bin created above with NVRAM variables listed in  nasoc/src/shared/nvram/bcm94780nas.txt embedded in the image.  This has all the basic NVRAM variables set with the hardware MAC address set to the default value of 00:10:18:EC:00:00 for Ethernet port 0 and 40:10:18:EC:00 for Ethernet port 1.

Important Note: Neither of these images (cfe.bin and cfenas.bin), should be programmed as the boot loader images for the BroadNAS.  cfe.bin does not have any embedded NVRAM variables and cfenas.bin is a default reference image only and does not have unique MAC addresses and other unique system-specific information.

### Creating a board speciic CFE

To create a board-specific CFE boot loader image, execute the nvserial utility with a custom text file that contains NVRAM variables with board-specific values. An example of such a text file can be found in nasoc/src/shared/nvram/bcm94780nas.txt.  The resulting CFE image can then be programmed as the final CFE boot loader on the board.

The syntax for nvserial is shown below:

% nvserial -i cfe.bin -o cfenas.bin -s <board num> bcm94780nas.txt

where 'board num' gets translated to lower 2 bytes of the Ethernet MAC address.

The cfenas.bin created with this command can be programmed as the boot loader.

The second option is to create and program a board-specific CFE via the automated flashcfe program.

Note: flashcfe is a TCL/Expect script program that runs under Linux.  You will need TCL and Expect installed on your Linux host PC.

Use the flashcfe program for programming the final boot loader.  Use the manual process of creating the board-specific CFE boot loader only if you are programming it on a blank Flash.

## Accessing CFE from the Console

### Console Settings

BroadNAS console settings for CFE and for the run-time firmware are the same.  The settings to be used for the console (Example. hyperterm on Windows or minicom on Linux) are as under –

Baud Rate – 115200

Number of data bits – 8

Parity - None

Number of stop bits – 1

Hardware Flow Control – No (Broadcom BroadNAS supports hardware flow control, and you can set that to yes if you want)

Software Flow Control – No

### Getting CFE Prompt

BroadNAS, by default, autoboots the firmware image and does not give the user an option to break into CFE.  To get to the CFE prompt, follow the steps below.

- Press and hold CTRL-C and power the BroadNAS system ON.

- Once you get the CFE prompt, modify the boot_wait NVRAM variable as under

  - nvram set boot_wait=on

  - nvram commit

  - reboot

- On reboot, you should see the following message on the console

Waiting to load image on IP <IP address of your system>, ^C to abort...

- This will allow you about 5 seconds to break into CFE by pressing CTRL-C before booting the firmware.

Note: If boot_wait is already defined as on, steps 1 and 2 above are not required.

## Programming CFE boot loader on a blank flash

In case of a blank Flash, the following 2 methods can be used to program the CFE boot loader image.

### Blank Flash NOT soldered on the board

- Create a cfenas.bin image with the nvserial utility as mentioned above with a board-specific NVRAM variable text file.

- Program the flash with this raw binary image via an EPROM/Flash programmer and then solder the Flash part on the board.

### Blank Flash or Flash with non-working CFE soldered on the board

- Create a cfenas.bin image with the nvserial utility as mentioned above with a board-specific NVRAM variable text file.

- Program this image on the on-board Flash using the MAJIC JTAG controller from Embedded Performance Inc (EPI).  Please refer to the Using the EPI Flash Programming Utility application note from EPI for details on how to do this.

#### Ordering MAJIC JTAG Controller From EPI

To order MAJIC JTAG from EPI contact them at

Embedded Performance, Inc.  606 Valley Way, Milpitas, CA 95035

Telephone: (408) 957-0350 FAX: (408) 957-0307

e-mail: support@epitools.com, web: www.epitools.com

Order the MAJIC kit for MIPS with EDB debugger and MAJIC 10-pin JTAG cable kit (CK-J10).  This comes with the software required to program the on-board Flash.  Refer to the Using the EPI Flash Programming Uitlity application note for detailed instructions on how to program the on-board Flash.

## Updating CFE boot loader

If a version of CFE exists in Flash and needs to be updated, the following 2 methods can be used.

### Updating Existing CFE via command line

- Create a cfenas.bin image with the nvserial utility as mentioned above with a board-specific NVRAM variable text file.

- On the host computer, run a program to download cfenas.bin to the BroadNAS.  The choice of program to run depends on the value of the ‘cfe_password_protect_tftp’ NVRAM variable.  If ‘cfe_password_protect_tftp’ is set to ‘enabled’, then run ‘nasload’ as follows:

> nasload –f <file to download> <IP address of BroadNAS>

If ‘cfe_password_prtect_tftp’ has any other value, use tftp to download cfenas.bin.  For Windows, use the following command:

> tftp –i <IP address of BroadNAS> put cfenas.bin

For linux, execute the following:

> tftp <IP address of BroadNAS>

tftp> bin

tftp> put cfenas.bin

- On the BroadNAS CFE console run

CFE> flash –noheader : flash0.boot

cfenas.bin will be TFTPed to BroadNAS and programmed in Flash.

### Updating Existing Firmware via command line

This part is mentioned here because of the logical reference.

- Break into CFE while booting by continuously pressing CTRL+C. Advise keeping CTRL+C pressed all the time while booting .

On the host computer, run a program to download broadnas.nas to the BroadNAS.   For Windows, use the following command:

> tftp –i <IP address of BroadNAS> put broadnas.nas

For linux, execute the following:

> tftp <IP address of BroadNAS>

tftp> bin

tftp> put broadnas.nas

3.	On the BroadNAS CFE console run

CFE> program :

For a non-split image this is sufficient and box will automatically reboot.

On a split image this will only program the flash. After reboot BroadNAS will launch pivot which will wait for firmware download again for the disk portion of the firmware.  This can be accomplished by doing the following on the host

>nasload –f <file to download> <IP address of BroadNAS>

### Updating Existing CFE via automated script and Serial Console

If a common CFE boot loader has been pre-programmed in the Flash, it needs to be updated during manufacturing with board-specific boot loader.  The board specific image has the unique MAC address and system specific options set.  To do this run the flashcfe program provided in the nasoc/src/tools directory.  This program requires nvserial, a NVRAM variable text file and the raw cfe.bin files.  The program prompts for system options, such as WLAN hardware availability, and for board specific MAC addresses.  For common values, edit the NVRAM variables text files.

- Copy nvserial, flashcfe, cfe.bin, and bcm4780nas.txt (or vendor specific file) in one directory on a Linux host PC.

- Edit the bcm4780nas.txt if require to modify any of the configuration settings.

- Connect the PC serial port to the BroadNAS serial port.

- Open a console program (minicom), turn the BroadNAS on and break into the CFE boot loader (CTRL-C).

- Exit from the console program.  It is required that you exit from the console program (minicom) for the flashcfe to work properly.

- Connect BroadNAS on the same network as the host PC.

- Run the flashcfe program.

- The program will prompt you for the serial console setup information first.  Set the console device (/dev/ttyS0 for COM 1) and leave the baud rate to default value of 115200.

- Next, the program will prompt for the board name.  This has to match the configuration text file.  Default text file is bcm94780nas.txt and hence the default board name is bcm94780nas.

- Next the program prompts for target IP address and netmask.  Make sure to set this so that the host PC can access the BroadNAS over Ethernet.

- Make sure you enter a board number when prompted for.  Not entering a board number will make you exit from the program.

- The next important information that the program prompts you for is the MAC addresses.  Once that is set the program will automatically update the CFE bootloader on BroadNAS.

- Ignore the messages till you get a prompt for board number again.  At this point you can program another board by entering a new board number or exit by just hitting Enter.

- Press the RESET button on BroadNAS and hold if for about 5 seconds to restore factory defaults if required.  Note that this is the hardware reset button and on the Broadcom BroadNAS reference platform it is the small white button on the board.

### Updating Existing CFE via automated script – No Console

CFE can be upgraded without using a serial cable. To this, the customer will need to write a special application that runs on the box and performs CFE upgrade if needed. Customer can use an NVRAM variable to record CFE version number. The application can use this NVRAM variable to find out if CFE needs to be updated. This application should go in a special image that can upgrades the CFE. By default, CFE partition of flash is read-only. This special image  will make CFE read/write.

Here is what this image/app needs to do:

- This new image includes special app and our trx.nas & nvserial.nas utlities.    This image should also include the default NVRAM variable file (everything  except board specific  parameters like MAC address).

- The new image will need to use reconfigured kernel that opens CFE  as read/write. This can be done  by changing the following in nasoc/src/linux/linux/arch/mips/brcm-boards/bcm947xx/ setup.c

from

{ name: "cfe",  offset: 0, size: 0, mask_flags: MTD_WRITEABLE, },

to

{ name: "cfe",  offset: 0, size: 0, }

- The application finds out the MAC address of the board and generates  the complete NVRAM variable file.

- The application runs nvserial.nas to generate cfenas.bin see section above

- The application runs trx.nas and burns the new CFE:trx.nas -f noheader -o cfenastrx.bin cfenas.bin write cfenastrx.bin cfe

## NVRAM variable text file

The CFE boot loader image that gets programmed in the BroadNAS Flash requires some basic NVRAM variables embedded in its image.  Some of the variables are mandatory and some are optional.  Also, some variables are unique per board/system and some define the system capability.  Listed below are the NVRAM variables in this file, their functionality, and their values for BroadNAS system.

Important Note: The base address for et0macaddr and et1macaddr is vendor specific and needs to be modified before running the flashcfe program.

### Required NVRAM variables

boardtype=0x042f

Description: Identifies the reference board type, so that the software can distinguish between different reference boards

boardnum=<unique value per board>

Description: Identifies the board number.

boardrev=0x10

Description: Revision of the board.  Used to distinguish different revisions of a given board type.

boardflags=0

Description: Flag to identify the availability of different options on a board.

clkfreq=264

Description: MIPS clock frequency in MHz

sdram_init=0x0009

sdram_config=0x0062

sdram_refresh=0x0000

sdram_ncdl=0

Description: Memory controller setup.  Current values shown are for 64 MB DDR SDRAM (16 Meg x 16 x 2) device on the Broadcom BroadNAS reference system.  Please contact Broadcom if you use different DDRs.

et0macaddr=00:10:18:00:${maclo}

et0phyaddr=5

et0mdcport=0

Description: Ethernet MAC setup for Ethernet port 0 of the BCM4780P.  This is the main LAN Ethernet port.  nvserial utility requires the et0macaddr defined as shown above, with the last 2 octets set by the serial number variable.

et1macaddr=40:10:18:00:${maclo}

et1phyaddr=30

et1mdcport=1

Description: Ethernet MAC setup for Ethernet port 1 of the BCM4780P.  BCM94780NAS does not have a second PHY.  Hence, this is pseudo setup.  MAC address of 40:... -> local address.  Setup is done exactly the same way as for Ethernet 0.

watchdog=0

Description: Watchdog timer in ms.  Value of 0 means that watchdog is disabled.

reset_gpio=7

Description: GPIO pin number tied to Reset for restoring factory defaults.

dl_ram_addr=a0001000

os_ram_addr=80001000

os_flash_addr=bfc40000

scratch=a0180000

boot_wait=off

Description: Different memory addresses used by CFE.  boot_wait is the option to give user the chance to break into CFE.

lan_ipaddr=192.168.1.1

lan_netmask=255.255.255.0

Description: Variables used by CFE and run-time OS image to setup LAN IP interface

lan_ifnames=eth0 eth2

lan_ifname=eth0

wlan_ifname=eth2

wan_ifname=eth1

Description: Variables used only by the run-time OS image to configure network interface names, but never modified by the run-time OS image

### Optional NVRAM variables

wlan_hardware_present=no

wlan_ipaddr=192.168.21.1

wlan_netmask=255.255.255.0

Description: Variables used only by the run-time OS image to setup WLAN IP interface

default_physical_authentication_enable=enabled

default_http_username=

default_http_passwd=nasoc

default_workgroup=WORKGROUP

default_start_page=/setup/initial_setup.asp

default_new_disk_action=no_action

default_primary_pool_name=

default_primary_share_name=

Description: Variables used by the run-time OS image to set up factory default values of variables that may be modified by the run-time OS image

Note - DO NOT change the prefix "default_".

# BroadNAS Software Debugging

There are three debugging methods of debugging BroadNAS software during development.

- The Embedded Performance Inc. (EPI) Development Kit for MIPS architecture using the MAJIC JTAG probe

- BroadNAS Console

- Syslog

## EPI Development Kit

The EPI development kit provides support for the BCM4780 based system via the MAJIC JTAG probe.  It consists of two components that communicate through an Ethernet port or a high speed RS-232C serial data port:

- debugger running on a host computer (EDB under Windows)

The debugger implements high-level debug functions by sending debug service requests to the MAJIC probe over the communication link. It then formats and displays the data or status information that is returned from the MAJIC probe

- MAJIC JTAG probe

The MAJIC probe contains the hardware and software that controls the JTAG interface to the processor’s Debug Support Unit (DSU), and performs debug services such as memory test, single stepping, and breakpoint management.

### Setting up EPI Debugging Environment under Windows

Please refer to the Debug Environment chapter in the MAJIC User’s Manual from EPI for details of setting up the EPI debugging environment.  This section gives some BroadNAS specific steps to be taken while setting up the EPI debugging environment.

Use the MAJIC Setup Wizard provided with the EPI toolkit.  Listed below is specific setup to be done while setting up the debugging environment for BroadNAS.

- Choose your debugger.

Select EDB as the debugger.

- Choose your processor.

Select BCM4780 as the processor if listed.  If BCM4780 is not listed, choose BCM4704.  Select the target endianes as little endian.

- Choose the method used to connect to your MAJIC.

Select the mehod you pick to connect to your MAJIC probe – serial RS232 or network.  If you select network, specify the IP address assigned to the MAJIC probe.

- Startup File

Select the “Use Existing Startup File” option.  Set the startup file as startice.cmd from the targets\bcm94780nas directory in the EPITools installation directory or from the targets\bcm94704agr directory if bcm4780nas is not available.

- Let the setup wizard create an icon on the desktop for EBD with your project name.

### Using EDB Debugger

EDB debugger can be used for downloading and debugging any program on the BroadNAS.  For Linux kernel debugging, build an image with debug option ON.  Setting the debug option ON for the entire kernel creates a very big image which takes a long time to download via JTAG.  We recommend turning the debug option ON only for modules that need to be debug.  Once you have the program to be debug built, do the following.

- Create an EDB setup configuration as described in the section above.

- Connect the MAJIC JTAG probe to the BroadNAS board JTAG connector (J46) taking care of the polarity.

- Power up the BroadNAS board and power up the MAJIC probe.

- Start the debugger by double clicking the shortcut created by the wizard.

- Now you can download a program and perform debugging actions like setting breakpoints, stepping through code, watching variables, etc.

Note:	The program to be downloaded needs to be a COFF or ELF image.  The downloaded program can be executed with no boot loader or any other boot program present in the boot Flash.

Note:	Please refer to the section on programming blank on-board Flash with CFE boot loader for information on ordering EPI.

## BroadNAS Console

The BroadNAS provides a console with a shell that can be very useful for development and debugging.  The console lets the developer see the progress of the software via the log messages and can also be used for running standalone programs and utilities.  It also enables the developer to access the /dev and /proc file system which can provide valuable system information.  All printfs from the application program are displayed on the console.  The kernel printks displayed on the console depend on the console_loglevel NVRAM variable setting.  If the console_loglevel is set to 10 all printks are visible on the console.

## Syslog

Syslog is an industry standard protocol used for capturing log information from devices on the network.  BroadNAS provides support for sending log messages over the network via the syslog protocol.  All syslog messages from the application programs and all kernel messages are sent to a remote host running a syslog server.  To enable sending syslog messages from BroadNAS, set the NVRAM variable log_ipaddr to the IP address of the syslog server.  This can also be done via the web GUI from the Administration/Alerts and Logging page.  Once syslog is enabled on BroadNAS, it will keep sending syslog messages to the server.  To enable the syslog server daemon on a RedHat Linux distribution, do the following.

- As root change /etc/sysconfig/syslog to add "-r -x" params to the SYSLOGD_OPTIONS environment variable.

- Then, as root, run /etc/rc.d/rc3.d/S12syslog restart.  From then on, anything logged should show up in /var/log/messages, depending upon priorities and the mappings in /etc/syslogd.conf.

Note:	The steps to enable the syslog server daemon may vary from system to system.

# Loading BroadNAS Without Programming Flash

## Introduction

During software development for BroadNAS, it is not required to burn the run-time kernel image and the root filesystem into Flash every time.  There are a few ways to avoid burning the image into Flash every time and speed up the development cycle -

- The kernel image can be loaded via TFTP (or nasload) from the CFE prompt.

- The root file system can be loaded via TFTP (or nasload) into RAM from the CFE prompt.

- The root file system can be mounted via NFS

This chapter deals with 1 and 2 above.

If NVRAM variable ‘cfe_password_protect_tftp’ is set to ‘enabled’ then you will need to run nasload instead of TFTP on the host computer.

## Required Setup

Listed below are some things that you need to boot the kernel image via TFTP (or nasload) and/or load and use the root filesystem in RAM.

- TFTP server running on a host PC that can be reached by the BroadNAS from CFE prompt.

- Console connected to the BroadNAS.

- Zipped kernel image “vmlinuz” and the target cramfs root filesystem file, “target.cramfs”.  Both these files can be found in your NASoC source ‘apps/mipsel’ directory.  You need to copy these to the TFTP root directory of your TFTP server.

- BroadNAS release 1.1 or later version.

## Loading CRAMFS Root Filesystem in RAM via TFTP

By default, the kernel is set to look for the cramfs root filesystem in the Flash.  If you want the kernel to look for the root filesystem in RAM, do the following.

- Set the following NVRAM variables.  Both these NVRAM variables need to be set to load and use the root filesystem in RAM.

  - rootfs_in_ram=yes

  - kernel_args=“root=/dev/mtdblock5 noinitrd console=ttyS0,115200 mem=56M”

- Copy the target.cramfs file from your source tree (src/apps/mipsel/target.cramfs) to the TFTP server root directory.

- Break into CFE and execute the following command –

load -addr=0x83800000 -max=0x800000 -tftp <TFTP  server IP address>:[<path>/]target.cramfs

‘path’ is optional if you have copied the file in a subdirectory under the TFTP root

- Type ‘go’ at CFE prompt to execute the kernel from Flash using the root filesystem loaded in RAM.  If you want to boot using a different kernel, follow the instructions in Loading and booting kernel from RAM below.

- To revert back to use the root filesystem in Flash, unset the kernel_args NVRAM variable or set root=/dev/mtdblock2.

Note: The values used for mem in kernel args and addr in the load command are for a system with 64MB RAM (eg. Broadcom BroadNAS reference system).

## Loading and booting kernel from RAM

Irrespective of loading and using the root filesystem in Flash, BroadNAS can boot with kernel loaded directly in RAM via TFTP (or nasload).  Here are the steps to do that.

- Copy the vmlinuz file from your source tree (src/apps/mipsel/vmlinuz) to the TFTP server root directory.

- Break into CFE and execute the following command –

boot –z –addr=0x80001000 –max=0x800000 –tftp <TFTP server IP address>:[<path>/]vmlinuz

‘path’ is optional if you have copied the file in a subdirectory under the TFTP root

NOTE: If loading both, the root filesystem and kernel image in RAM via TFTP, load the filesystem first and then boot the kernel.

# Customizing GPIO’s

BroadNAS system has a number of miscellaneous inputs and outputs such as LEDs, buttons, switches, etc.  These inputs and outputs may be controlled via an external device such as a CPLD or via the general purpose inputs and outputs (GPIO) of the BCM4780P processor.  BroadNAS application software provides support for a set of pre-defined inputs and outputs.  The CPLD on the Broadcom reference BroadNAS provides hardware support for controlling these pre-defined inputs and outputs.

Different hardware implementations of BroadNAS may have different configuration of the miscellaneous inputs and outputs.  Also, these may be controlled via a different external device or via the BCM4780P GPIO.  This chapter describes how the miscellaneous inputs and outputs on the Broadcom BroadNAS reference system is controlled using the Linux proc file system.  It also describes how the control of miscellaneous inputs and outputs can be customized for different hardware configurations via the BCM4780P GPIO using the Linux proc file system.

Note:	Customized control of miscellaneous inputs and outputs is available only in BroadNAS software release 1.1 and later.

## Introduction

### BroadNAS System Miscellaneous Inputs and Outputs

BroadNAS application software supports the following miscellaneous input and outputs via the Linux proc file system.  On the Broadcom BroadNAS reference hardware these are controlled via a CPLD which is interfaced to the BCM4780P via four GPIO lines.

#### Inputs

| Entry | Description |
| --- | --- |
| soft_power_switch | Shut down services to prepare for power down. Active high. |
| authentication_button | Physical authentication for web access. Active high. |
| temperature_alert | Input from external temperature sensor device. Active low. |
| drive_0_eject_request | Request to eject the first drive. Active high. |
| drive_1_eject_request | Request to eject the second drive. Active high. |
| drive_2_eject_request | Request to eject the third drive. Active high. |
| drive_3_eject_request | Request to eject fourth drive. Active high. |

#### Outputs

| Entry | Description |
| --- | --- |
| system_ready_LED | LED to indicate system is ready with all services started. 0 – Off, 1 – On |
| drive_0_reset | Software control driver reset for primary IDE drive(s). 0 – Reset Drive, 1 – Normal operation |
| drive_1_reset | Software control driver reset for secondary IDE drive(s). 0 – Reset Drive, 1 – Normal operation |
| power_enable | Enable ATX power. 0 or 1. |
| twin_copy_LED | LED to indicate twin copy. 0 – Off, 1 – On. |
| drive_0_status_LED | LED to indicate status for the first drive. 0 – Off, 1 – On. |
| drive_0_removeOK_LED | LED to indicate if it is okay to remove the first drive after eject request button is pressed. 0 – Off, 1 – On. |
| drive_1_status_LED | LED to indicate status for the second drive. 0 – Off, 1 – On. |
| drive_1_removeOK_LED | LED to indicate if it is okay to remove the second drive after eject request button is pressed. 0 – Off, 1 – On. |
| drive_2_status_LED | LED to indicate status for the third drive. 0 – Off, 1 – On. |
| drive_2_removeOK_LED | LED to indicate if it is okay to remove the third drive after eject request button is pressed. 0 – Off, 1 – On. |
| drive_3_status_LED | LED to indicate status for the fourth drive. 0 – Off, 1 – On. |
| drive_3_removeOK_LED | LED to indicate if it is okay to remove the fourth drive after eject request button is pressed. 0 – Off, 1 – On. |

### BCM4780P General Purpose Inputs and Outputs

The BCM4780P processor has eight general purpose input/output (GPIO) pins.  Either of these can be configured as an input or an output.  Registers are available to configure the pins as either input or output, to read the current bit value on the input pin or read the last bit value written on the output pin, and to write a bit value to the output pin.  Registers are also available to set up interrupt mask to generate interrupt on an input and to define the polarity of the input for which the interrupt should be generated.

## Hardware Configuration

### Using CPLD

The miscellaneous inputs and outputs can be controlled via different hardware.  The Broadcom BroadNAS reference design uses a CPLD to control the BroadNAS system miscellaneous inputs and outputs.  The CPLD interfaces with the BCM4780P via five GPIO lines as described below.

GPIO0 - CPLD Clock (SCL) - output

GPIO1 - Chip Select (CS1) - output

GPIO2 - Serial bit stream from the host to the CPLD (SDO) - output

GPIO3 - Serial bit stream from the CPLD to the host (SDI) - input

GPIO4 - CPLD interrupt to the host (SINT_L) – input

The application software accesses the miscellaneous inputs and outputs via the proc file system.  The lower level support for the proc file system is provided in the CPLD driver.  The inputs and outputs available via the CPLD are listed in section 3.1 above.

### Using 4780P GPIO Pins

The BCM4780P GPIO pins can be used directly to access miscellaneous inputs and outputs.  The application software access to the GPIO controlled miscellaneous input and output is also available via the proc file system.  The lower level support for the proc file system in this case is provided in the GPIO driver.

Important:	Since the BCM4780P has only eight GPIO pins and one of them is dedicated for system Reset/Factory Default, the number of miscellaneous inputs and outputs available in this case is only seven.

Note:	Since the BCM4780P accesses the CPLD via GPIO, both discrete GPIO control and CPLD control of the miscellaneous inputs and outputs is not available together.  That is, if CPLD is used, then discrete GPIO cannot be used and vice versa.  Although only 5 GPIO lines are used for CPLD access, providing control of GPIOs along with CPLD causes contention and hence this restriction.

A different device and/or a different interface can be used instead of the CPLD for controlling the miscellaneous inputs and outputs.  The user level application via the proc file system can still be used in this case, but a new driver with lower level support for the proc file system will be required.

### Selecting the hardware configuration

Device drivers for the CPLD and GPIO are provided by Broadcom. These device drivers are loaded as kernel loadable modules.  An NVRAM variable (misc_io_mode) is defined to select which hardware configuration is being used and hence which device driver is loaded.

A custom device driver can also be written to provide support for any other custom hardware.

## Software Configuration

### Broadcom BroadNAS Reference Platform

On the Broadcom BroadNAS reference platform, the miscellaneous inputs and outputs are controlled via an external CPLD.  BroadNAS application software accesses the miscellaneous inputs and outputs listed in section 3.1 above, via the proc file system.  If the misc_io_mode NVRAM variable is not defined or defined as bcmmiocpld, the CPLD driver will be loaded and used by the proc file system to access the miscellaneous inputs and outputs.  A proc file entry corresponding to each input listed in section 3.1.1 and each output listed in section 3.1.2 is available under /proc/miscio in the root file system of NASoC.  Each of the input entry can be opened to read the latched bit value and clear the latched bit value.  Each of the output entry can be opened to set the output bit value.  The polarity of each input and output is also listed in section 3.1 above.

For hardware based on the Broadcom BroadNAS reference platform, using the CPLD, the only customization available is to use any of the inputs or outputs listed in section 3.1 for some other purpose.  For example, the power_enable output can be used to turn a fan on or off.  The thing to keep in mind while doing such customization is that the BroadNAS application software will monitor the input and control the output assuming the functionality listed in section 3.1.  Considering the same example of the power_enable output for fan control, the BroadNAS software will assert or de-assert the signal when ATX power has to be turned on or off and hence the fan will be turned on or off at that time.

### Customizing miscellaneous inputs and outputs for other hardware configuration

This section describes how the miscellaneous inputs and outputs access can be customized for different hardware configuration.  Discrete GPIO is used as an example as driver support for GPIO is provided in the NASoC software.  A similar driver will have to be provided for any other hardware configuration.  Listed below are the customization steps.

#### Provide a device driver

Provide a device driver for the hardware device that will be used to access the miscellaneous inputs and outputs.  The device driver should be built as a kernel loadable module.  The driver should create entries under the /proc/miscio directory for reading or manipulating the bit value of the input or output.  Additional entries can also be created to configure the input or output.

For GPIO:

bcmgpio.o is the kernel loadable device driver module provided for using discrete GPIO pins as miscellaneous inputs and outputs.  The process for loading the driver, and the entries created by this driver under /proc/miscio and its usage is described in detail in the following sections.

#### Select and load the device driver

The hardware configuration and hence the device driver to be loaded is selected by means of the misc_io_mode NVRAM variable.  During system initialization, the system software will check for the value of misc_io_mode and load the appropriate device driver.

For GPIO:

misc_io_mode defined as bcmgpio will load the GPIO device driver.  If misc_io_mode is not defined or defined as bcmmiocpld, the CPLD device driver will be loaded.  Hence it is important to define the misc_io_mode as something if you don’t want the system to load the CPLD device driver by default.

#### Entries created by the device driver

The device driver will create file entries corresponding to the inputs and outputs supported by the device driver under the /proc/miscio directory.  The entries provide a mechanism of reading the input bit value or setting the output bit value.  The application software opens these files for read and/or write.  Reading the file may return the current bit value as a string (‘0’ or’1’).  String representation of the bit value may be written to the file to manipulate the output.  The actual values that can be read or written and their interpretation is dependant on the device driver. Additional file entries can be provided to configure the inputs and outputs.  The entries supported by the GPIO driver, and their usage is described below.

For GPIO:

The GPIO driver provides two entries per GPIO pin under the /proc/miscio directory.  The two entries provided per GPIO pin are as under –

gpio_X_config	- where X is GPIO pin number 0-7. This entry is used to configure the GPIO pin.  The following configuration parameters are available –

direction	- Sets the GPIO pin as input or output.  To configure the direction of the GPIO pin, write the following string

direction=in 	To set the GPIO pin as input (default configuration)

direction=out	To set the GPIO pin as output

polarity	- Sets the output polarity (mainly used for LEDs which is reverse, i.e. writing 0 turns it ON, and writing 1, turns if OFF).  To set the output polarity, write the following string

polarity=normal 	To set normal polarity (default configuration)

polarity=reverse	To set reverse polarity, where writing the value of ‘1’ will set the GPIO pin to ‘0’

interrupt	- Set whether the value on the input pin is latched via an interrupt or not.

interrupt=no	If the pin is not configured to be interrupt driven, the application reads the current value of the pin (default configuration)

interrupt=yes	To make the input pin interrupt driven.  In this case, the value of the input pin will be latched by the driver when an interrupt occurs and needs to be cleared by the application software.

interrupt level	- Sets the polarity of the input signal on which an interrupt is to be generated if the input pin is configured to be interrupt driven.  To set up the interrupt level, write the following string

interrupt level=1	Active high interrupt (default configuration)

interrupt level=0	Active low interrupt

Note:	The above configuration setup strings are case sensitive and should have no spaces before or after the ‘=’.  The string needs to be written to the /proc/miscio/gpio_X_config file, where X is the pin number to be configured (or to the BoadNAS specific I/O, config file – see the section below on mapping for explanation).

gpio_x	- where ‘x’ is GPIO pin number 0-7.  This entry is used to read the input value or write the output value.  For a GPIO pin configured as input with interrupt enabled, reading this file will return the latched value string which may be ‘0’ or ‘1’.  The latched value needs to be explicitly cleared by the application software by writing ‘0’ to the file entry.  For input pin which is not interrupt driven, reading this file will return the string representation of the current bit value on the corresponding GPIO pin.  For GPIO pins configured as output, ‘0’, ‘1’, or ‘2’ can be written to the file entry.  If the pin is configured with reverse polarity, writing ‘0’ to the file will set the GPIO pin high and writing ‘1’ to it will set it low.  For normal polarity, writing ‘0’ will set the pin low and writing ‘1’ will set it high.  Writing ‘2’ to the file for output GPIO pin will toggle the value on the GPIO pin between ‘0’ and ‘1’ every 500msecs.  This is used for blinking LEDs.

Note:	If the GPIO pin is mapped to an input or output listed in section 1.1, the entries will be for that name rather than “gpio_x”.  Please refer to the section below for details.

Note:	If a SMBUS device (eg. RTC or Temperatur controller) is used which is controlled over 2-wire interface by bit banging via the GPIO, 2 of the GPIO pins will be dedicated for that.  In that case, those 2 GPIO pins should NOT be used as discrete I/Os and the entries for those pins under /proc/miscio should not be accessed.  Currently, GPIO pins 2 and 5 are reserved for SDA and SCL respectively.

Important:	GPIO pin 7 is dedicated for Reset/Factor defaults and hence should NOT be used as discrete I/O.  Although the /proc/miscio directory has entries for gpio_7 and gpio_7_config, that should not be used.

#### Mapping device I/O to BroadNAS system inputs and outputs

Any of the inputs and outputs supported by the custom device can be mapped to any of the input listed in section 3.1.1 or output listed in section 3.1.2.  This can be done by defining a NVRAM variable making the driver add an entry for the mapped name.  This will allow the BroadNAS software application to control the miscellaneous inputs and outputs listed in section 1.1 via the custom hardware.

For GPIO:

The NVRAM variable for mapping the BroadNAS system input or output to a GPIO pin is gpio_X_name, where X is the pin number.  For example, if GPIO pin 0 is to be mapped to authentication_button, a NVRAM variable, gpio_0_name will be defined as “authentication_button”.  Make sure that the pin is configured properly before they ar used.  In this case, the entries for gpio_0 and gpio_0_config in the /proc/miscio directory will be replaced with authentication_button and authentication_button_config respectively.

# Customizing Defaults

## Customizing Default Disk & Printer Actions

The NVRAM variable is

"new_disk_action" which by default is set to "no_action".  To set default behavior to be something else, set "default_new_disk_action" in the NVRAM text file to one of the following - "no_action", "one_jbod", "one_mirror", "one_per_disk".

If `no_action' is selected, then the BroadNAS will not do anything with the disk on its own, it will wait for user to go into the web configuration interface and specify how the disk is to be used.

If `one_jbod' is selected, then when a new disk is added, if the primary pool does not already exist, it will be created across the entire disk and the primary share will be created within that pool.  BroadNas software will use primary_pool_name and primary_share_name nvram variables. If the primary pool already exists when a new disk is added, then the primary pool will be expanded to encompass whatever it already encompassed plus the entire new disk.

If  `one_mirror' is selected, then if the primary pool doesn't already exist when a disk is added, the primary pool will be created on the entire disk.  If the primary pool already exists on one disk when a new disk is added, the new disk will be used to mirror the primary pool if it is at least as big as the original pool.  If the new disk is too small or if the primary pool is already mirrored, then no action will be taken and user must use the web configuration interface to manually tell the BroadNAS how to use the new disk.

Finally, if  `one_per_disk' is selected, then each time a new disk is added, a pool will be created for that disk that spans that entire disk and only that disk and a corresponding share will be created in that pool.

For anything other than 'no_action', 'primary_pool_name' and 'primary_share_name' has to be defined.  Again, by default this has no values.  One can define 'default_primary_pool_name' and 'default_primary_share_name' in the text file.  These values will be used as the names of the pool and the share when new ones are created.

For printer pool creation, the NVRAM variable is "print_spool_auto_create". When set to "enabled", the printer spool is automatically setup on a new disk that is claimed as part of default disk action.

## Extend network nvram variables functionality

The following are the steps to extend the functionality of network nvram variables. Currently only "wan_proto" can be extended.

### Adding new functionality for wan_proto

If you want to add a new value for wan_proto, say ppptp, that is not supported by Broadcom, follow these steps :

1. Implement start and stop functions for ppptp that are invoked when the wan interface comes up or shuts down. These functions take a void pointer that gets passed from wan network services.

2. Write a *_dynamic_init() function that will be automatically called. The "*" represents the library name that you provide for extending the wan_proto functionality. So if the library name is libsamplenetregistration the init function will be samplenetregistration_dynamic_init().

3. Within this function call register_net_mode with four arguments. The first argument will be the nvram variable name, the second will be the nvram value, the third will be the user visible value for the value, the fourth will be the start function pointer for that new value and fourth will be the stop function pointer.

So for example to register a new value "ppptp" for "wan_proto" the call will be register_net_mode ( "wan_proto", "ppptp", TRANSLATE("PPPTP"),  start_wan_ppptp,stop_wan_pptp);

The TRANSLATE macro is for internationalization purpose. start_wan_ppptp and stop_wan_ppptp will be function pointers that will be invoked at wan startup and shutdown (if wan_proto is set to "ppptp").

Note: See the sourcecode in apps/samplenetregistration/samplenetvarregistration.c to see how to register network nvram values.

4. To make sure the NASoC software picks up these changes, set the nvram variable "web_hook_libraries" as follows

"corewebhooks netwebhooks diskwebhooks winnet printwebhooks netregistration samplenetregistration"

### Overriding existing functionality for wan_proto

If you want to override the existing functionality for wan_proto, say pppoe, for which support exists already, follow these steps

1. Implement new start and stop functions for pppoe that are invoked when the wan interface comes up or shuts down. These functions take a void pointer that get passed from wan network services.

2. Write a *_dynamic_init() function that will be automatically called. The "*" represents the library name that you provide for extending the wan_proto functionality. So if the library name is libsamplenetregistration the init function will be samplenetregistration_dynamic_init().

3. Within this function call register_net_mode	with four arguments. The first argument will be the nvram variable name, the second will be the nvram value, the third will be the user visible value for the value, the fourth will be the start function pointer for that new value and fourth will be the stop function pointer.

So for example to override "pppoe" for "wan_proto" the call will be like register_net_mode ("wan_proto","pppoe",TRANSLATE("MY-PPPOE"), new_start_wan_pppoe, new_stop_wan_pppoe);

The TRANSLATE macro is for internationalization purpose. new_start_wan_pppoe and new_stop_wan_pppoe will be overriding function pointers that will be invoked at wan startup and shutdown (if wan_proto is set to "pppoe").

4. The key point to note for overriding functionality is the order of libraries in "web_hook_libraries". So in our case where pppoe functionality is provided by the new library should come after the default library.So set the nvram variable "web_hook_libraries" as follows

"corewebhooks netwebhooks diskwebhooks winnet printwebhooks netregistration samplenetregistration"

## Receiving Event Notifications

The summary strings are already attached in the User Manual Appendix.

Basically the strings are the summary of the event notification. They are:

- No Self Test Result

- Self test Passed

- Self test Aborted

- Missing Disk

- Disk Broken

- Disk Disk Name was removed unexpectedly

- Pool Pool Name is not yet mounted.

- Error: Temperature exceeded critical limit

- On getting any of these events BroadNAS firmware will put the string in /tmp/notification. String will be the same as listed in the attachment above.

  - If /tmp/notification is not present, it will be created.

  - String will be appended to end of file.

  - BroadNAS will hold a lock on the file before writing to it.

- Signal SIGUSR1 will be sent to a process whose name will be derived from nvram variable “event_process”.

- What applications receiving the signal should do:

  - Add program name using nvram set event_process="your program name"

  - In that program use an event handler for catching SIGUSR1.

  - On receiving event obtain a read lock on the /tmp/notification

  - Read the error string

  - -Parse and take appropriate action. More information on the error can be obtained from /tmp/user_notes.

### New Disk Arrival Notification

On New Disk Arrival notification BroadNAS software will

- Put the string in /tmp/notification. String will be the same as listed in the attachment above.

  - If /tmp/notification is not present, it will be created.

  - String will be appended to end of file.

  - BroadNAS will hold a lock on the file before writing to it.

- Signal SIGUSR1 will be sent to a process whose name will be derived from nvram variable “event_process”.

- What applications receiving the signal should do:

  - Add program name using nvram set event_process="your program name"

  - In that program use an event handler for catching SIGUSR1.

  - On receiving event obtain a read lock on the /tmp/notification

  - Read the notification string

Note: Disk name will just say SYSTEM. Application programs should use web hook

disk_pool_mapping_info to get the complete lists of disks. Sometimes a primary pool and share will also be created based on default disk action.

# Customizing Temperature sensor and Fan Devices

The BroadNAS reference platform is designed to use a temperature sensor device to monitor the temperature of the system and to control the speed of the fan based on the temperature reading.

The reference software is shipped with the device drivers for various devices, such as:

- lm63:  Temperature sensor with integrated fan controller

- g751:  Temperature sensor

- pwmfan:  BCM4785’s integrated fan controller

In addition, a generic Temperature Monitor and Fan Control Daemon (sensord) is provided to control the speed of the fan based on the temperature reading.  This sensor daemon is designed to operate with any sensor device.  This section describes how to configure the system to use a different sensor device.

## Device Drivers

### proc filesystem interface

To use a different sensor device, you have to provide the device driver for your particular device.  The device driver must support the following Linux proc filesystem interface:

| Entry | Access | Function |
| --- | --- | --- |
| fan_duty_cycle | Read/Write | Fan duty cycle in percentage. |
| temperature | Read Only | Temperature reading. |

Broadcom’s device drivers create the proc entries at the following directoies:

| Device | location of proc entries |
| --- | --- |
| lm63 | /proc/sys/dev/sensors/lm63-i2c-0-4c |
| g751 | /proc/sys/dev/sensors/g751-i2c-0-48 |
| pwmfan | /proc/sys/dev/pwm |

To use Broadcom’s sensord daemon, NVRAM variables sensord_temp_device_proc_dir and sensord_fan_device_proc_dir must be set indicating the location of the temperature and fan_duty_cycle proc entries, respectively.

### Loading device drivers

Temperature sensor and fan controller drivers are written as loadable modules and they are loaded at system startup.  The following nvram variables control which drivers are loaded:

| Name | Type | Default | Description |
| --- | --- | --- | --- |
| nasocflags | 32-bit flags |  | Bit 5 (i.e. mask of 0x00000020) indicates whether to load the temperature sensor driver. Bit 6 (i.e. mask of 0x00000040) indicates whether to load the fan controller driver. |
| fan_device | String |  | Name of the fan contoller device. The module to be loaded is bcm<fan_device>.o; where <fan_device> is substitued with the value of this nvram variable. |
| temp_device | String |  | Name of the temperature sensor device. The module to be loaded is bcm<temp_device>.o; where <temp_device> is substitued with the value of this nvram variable. |

## Temperature Monitor and Fan Control Daemon

A daemon is provided to monitor the temperature and control the fan.  The daemon provides the following functionalities:

- Control the speed of the fan based on the temperature reading.

- Send failure notification to user and shut down system when the temperature exceeds the critical setpoint limit.

### Auto Fan Control

The speed of the fan is automatically controlled over a specified temperature range.  When the temperature exceeds the High Temperature Setpoint, the fan will start and operate at a specified duty cycle.  When temperature reaches the end of the range, the fan will operate at 100% duty cycle.  Midpoint in the temperature range, the fan will operate at the median duty cycle.  The median duty cycle is halfway between the Low Duty Cycle and 100%.

If the temperature is above the high setpoint, then drops below the setpoint, the fan will remain ON until the temperature goes a certain amount below the setpoint.  The hysteresis value controls this amount.

100

Start

(100+start)/2

Hysteresis

Critical

Fan OFF

Fan ON

Temperature (°C)

Fan

Duty

Cycle

(%)

Auto Fan Control

0

High Setpoint

High Setpoint

+ Range

High Setpoint

+ (Range/2)

System

Shutdown

Figure 1. Auto Fan Control

When the temperature exceeds the critical temperature setpoint, error notification is sent and the system is shut down.

### sensord NVRAM variables

| Name | Type | Default | Description |
| --- | --- | --- | --- |
| sensord_poll_interval | Decimal | 5 | Polling interval. Number of seconds between temperature readings. |
| sensord_temp_high_setpoint | Decimal | 30 | Temperature high setpoint (in °C). When temperature exceeds this setpoint value, the fan will start. |
| sensord_temp_hysteresis | Decimal | 4 | Temperature hysteresis (in °C). The fan will not turn off until the temperature goes below sensord_temp_high_setpoint minus sensord_temp_hysteresis. |
| sensord_temp_high_range | Decimal | 20 | Size of the temperature window (in °C) in which the speed of the fan will change based on the temperature reading. |
| sensord_temp_skip_midpoint | Boolean | disabled | When set to "enabled", sensord does not define the mid-point temperature grid. Valid values: enabled ¦ disabled. |
| sensord_fan_dutycycle_start | Decimal | 50 | The initial duty cycle (in %) of the fan when the temperature reaches the high setpoint. |
| sensord_fan_kickstart_time | Decimal | 1 | Number of seconds the fan will run at full-speed before being set to the target speed. This is useful for fans that have trouble going from 0 to a low duty-cycle. |
| sensord_fan_control | Boolean | enabled | Indicate whether to control the speed of the fan. Valid values: enabled ¦ disabled. |
| sensord_temp_critical_setpoint | Decimal | 60 | Critical temperature setpoint (in °C). |
| sensord_fan_device_proc_dir | String | /proc/ sys/dev/ sensors/ lm63-i2c-0-4c | The location of the proc filesystem directory which contains the interface to the fan controller device. |
| sensord_temp_device_proc_dir | String | /proc/ sys/dev/ sensors/ lm63-i2c-0-4c | The location of the proc filesystem directory which contains the interface to the temperature sensor device. |
| sensord_let_die | Boolean | disabled | The daemon automatically restarts itself if it dies. Set this variable to “enabled” to not auto restart the daemon. Valid values: enabled ¦ disabled. |

# Small Flash Support

The NASoC software supports the option of using a small (2MB) flash device as a primary boot device for booting the kernel, while using an internal hard disk as a secondary boot device for booting the required root filesystem.

## Configuring an Existing System

If you have an existing system that is configured to put all of its software in flash and you wish to convert it to use the split image configuration, these are the steps you need to take.

Prerequisites:

- A BCM94780 system

- At least one IDE disk that has been claimed (NOTE: SATA and USB disks are not currently supported.)

- At least 16 MB of free space (not allocated for any pools) on one of the IDE disks.

Once you have a system meeting those requirements, you’re ready to convert it to using the split image system.  Follow these steps:

- Type “cat /proc/partitions” at the NASoC console.  Save the results somewhere for later reference.

- Use the respart program to create two reserved partitions on the IDE disk you will be using for the split image.  At this point you will need to decide how big to make each partition.  8MB per partition is enough for our current system software plus significant headroom, but since disks are so huge these days, you might want to consider using a much larger partition.  If you use 100MB for each partition, it will still be a tiny fraction of a modern disk but it will give you a huge amount of room for growth of the software.  You can type ``respart’’ at the NASoC console to run this program.  It takes two command-line arguments.  The first is the name of the disk.  This is the name it was claimed as – the name used in the web GUI.  It is not the raw device name.  The second parameter is the size of the partition in bytes.  For example, I might type

  - respart “My Disk” 100000000

to add a reserved partition of size 100 MB on the disk I named “My Disk”.  Run the command twice to create two reserved partitions.

- Type “cat /proc/partitions” again at the NASoC console.  Compare it to what you saw when you typed this before.  You should see two new entries, one for each of the two new partitions that you just created.  These are listed by device name but with the “/dev/” prefix omitted.

- Add “/dev/” to the start of each of the new entries in /proc/partitions to get the full device names for the two new reserved partitions.  They should be something like /dev/ide/host2/bus0/target0/lun0/part3.  Partitions should always end with “part<n>” for some integer <n>.  Now set the two NVRAM variables pivot_root_1 and pivot_root_2 to these two partitions.  For example:

  - nvram set pivot_root_1=/dev/ide/host2/bus0/target0/lun0/part3

  - nvram set pivot_root_1=/dev/ide/host2/bus0/target0/lun0/part4

  - nvram commit

- Set the pivot_root_current NVRAM variable to 1.

- Download the image broadnas_split.nas to your BroadNAS system.

When you’re done, the system should come up using less than 2 MB of flash for its software with the rest of the software on the disk.

The following sections describe in more detail what is going on when you use this setup.

## NASoC Image Components

In this scenario, the typical NASoC image is split into two separate software components:

- Boot Image - Kernel and minimal support drivers

The ‘Boot Image’ component of the NASoC image, consisting mainly of the kernel and the drivers required to configure the hardware necessary to boot the kernel. It is less than 2MB in size, and is burned into the flash memory boot device. After booting successfully, the ‘Boot Image’ will then locate and load (from disk) the ‘Rootfs Image’ of the NASoC image.

- Rootfs Image - NASoC root filesystem and applications

The ‘Rootfs Image’ component of the NASoC image contains the complete root filesystem for the NASoC, as well as all of the applications required for full NAS functionality. The size of the ‘Rootfs Image’ may be 4MB or greater, depending on the required feature set of the NASoC platform. The ‘Rootfs Image’ is stored on an internal hard disk, and is accessed by the ‘Boot Image’ after the kernel has successfully booted from the flash device.

## NVRAM Variables

The ‘Boot Image’ will access the ‘Rootfs Image’ according to the settings of the following NVRAM variables:

- pivot_root_current

= 0 - instructs the ‘Boot Image’ to claim 2 private partitions on the hard disk to store the ‘Rootfs Image’.

= 1 or 2 - instructs the ‘Boot Image’ to access and load the ‘Rootfs Image’ stored at the private partition indicated by the NVRAM variables pivot_root_1 or  pivot_root_2. This is referred to as the ‘current’ private partition.

- pivot_root_1 , pivot_root_2

Contain the fully defined paths to the private partitions where the ‘Rootfs Image’ is stored (i.e. /dev/ide/host1/bus1/…).

- pivot_root_wait

!= 0 – instructs the ‘Boot Image’ to try and load an updated ‘Rootfs Image’ from the network before loading it from the ‘current’ private partition. If a ‘Rootfs Image’ is successfully loaded from the network, it will be written to the ‘non-current’ private partition, and the ‘Boot Image’ will attempt to access it instead of the one located at the ‘current’ private partition. If the ‘Boot Image’ is not able to successfully load a ‘Rootfs Image’ from the network for a period of a minute or so, it will then attempt to access the ‘Rootfs Image’ from the ‘current’ private partition.

If the ‘Boot Image’ is unable to access a ‘Rootfs Image’ from the ‘current’ private partition, it will then attempt to access one from the ‘non-current’ private partition.

If the ‘Boot Image’ is unable to access a ‘Rootfs Image’ from neither the ‘current’ private partition nor the ‘non-current’ private partition, then the ‘Boot Image’ will attempt (indefinitely) to load a ‘Rootfs Image’ from the network.

## NASoC Rocovery

In normal operation, the NASoC software may be upgraded via the web interface.  However, if in a small flash device NASoC the boot image is not able to find the rootfs image (perhaps because the disk has been corrupted or replaced), a new rootfs image may be downloaded to the NASoC over the network.  This download will not upgrade the boot image, but that can be done, if desired, via the web interface once normal operation is restored.

To download a new rootfs, use the nasload program (see Upgrading Firmware with NASLOAD).  Nasload takes a NAS format file from which it extracts the rootfs image portion.  The extraction process is done by "nasunpack.exe".  Nasunpack writes out the rootfs to a temporary file on the Windows host; nasload deletes this file after the download is finished (successfully or not).

The design of nasload assumes that the OEM has bundled nasload in the same directory with nasunpack and the NAS file (e.g., broadnas_split.nas) and nasload.exe is started from that directory.  Therefore, nasload does not prompt for the NAS file location.

When invoked without any command line arguments, nasload can handle multiple ailing BroadNASs.  However, they will be handled serially.

# Downloading Firmware with NASLOAD

Users will normally upgrade the BroadNAS software via the web interface.  However the web interface may be inoperative if the firmware becomes corrupted.  In this case, the user must run a download program on a host computer in order to download new firmware.  Depending on the situation and NVRAM values, the download program may be a standard utility (i.e., tftp) or use a BroadNAS specific utility (e.g., nasload).

Broadcom provides an example implementation of a download program called nasload that uses the BroadNAS specific protocol.

## Discovery of NAS on the network

Some customers ship installation CDs that detect and configure a NAS after a customer has attached the NAS on the network.  Every BroadNAS box with comes CIFS support enabled and  registers the NetBIOS group name BroadNAS<bc> .  Any means may be used to enumerate the systems on a network.  One approach is to use the following command.  It lists the NetBIOS computer name of all computers on the network:

net view

And the following command to enumerate the registered netbios names for each system 

  nbtstat -A <computer-name> 

If the returned list of names includes BroadNAS<bc> then it is a BroadNAS device.  The netbios name with suffix 3 returns the IP address of the computer.

Once a NASOC has been detected, an agent residing on the NASOC can be used to communicate to the client PC configuration software to change the settings.

## When a Download Program is Required

There are three error cases that cause BroadNAS to look for a download program:

- The CFE code is corrupt.

- The run-time firmware in flash is corrupt.

- The run-time firmware on disk is corrupt (this is only for systems using Small Flash Support).

CFE detects cases 1 and 2, and the pivot boot image detects case 3.

The pivot boot image always requires the use of the special NAS upgrade protocol; whereas, CFE may or may not depending on the value of the cfe_password_protect_tftp NVRAM variable (see “Updating CFE boot loader”).  When cfe_password_protect_tftp is set to ‘enabled’, CFE requires the use of the NAS upgrade protocol; otherwise, CFE requires the use of a tftp client.  The NAS upgrade protocol requires that downloads be authorized but tftp does not.

## The NAS Upgrade Protocol

BroadNAS has its own protocol for downloading new firmware when the existing firmware has been corrupted.  In this protocol, the host side program is the server and the BroadNAS is the client.

### Sequence

This section describes the message flows in the BroadNAS specific NAS upgrade protocol.  There are two variants of the protocol with different mechanism for sending messages and different messages depending on the client type.  The two types of client are CFE and pivot boot image.

The protocol has three phases:

- Announcement: This is used to allow discovery of ailing BroadNAS devices on a local subnet.  If the IP address of the ailing BroadNAS is known (as much be the case if it is not on the local subnet), then the announcement may be sent to a specific IP address.  Announcements are sent on UDP port 16727.

- Authorization: An ailing client identifies itself to the server and (implicitly) requests authorization for the download.  The server must send an authorization request message with the administrator’s username and password.  The client verifies the username and password (against the contents of NVRAM) and then sends an authorization response with an indication of success or failure.

For CFE clients, authorization is done over UDP port 16728, and for pivot boot image clients authorization (and download) is done over TCP port 16728.

- Download: The file is massaged (if necessary) by the server and sent to the client.

For CFE clients, the download is actually done via the standard tftp protocol.  For pivot boot image clients, the file is sent across a TCP socket.  When sending the file across a TCP socket, the server first sends a data begin message with the file length.  The server then sends an arbitrary number of data messages with binary data.  The client acknowledges each data message (with an ack) and counts the number of bytes received.  When all bytes have been received it does a CRC check.

Errors may be detected on either the client or the server, and each has its own message type for reporting an error.  A BroadNAS client sends an error message which has a string describing the error condition.  The server sends an abort message which simply notifies the client that download attempt is aborted.

Additional information can be found in the nasload source files nasload.cpp and nasload.h.

### Message Format

The download program and BroadNAS exchange messages.  The messages have a header and a body.  The header format is shown below.  The contents of the body varies by message type and more detail is given in the comments in nasload.cpp

The fields in the message format are ASCII.  Each message header has the following format:

<tag> <sep> <type> <sep> <len> <sep>

The separator (sep) is the colon character.  The tag value is "BroadNAS".  The length field (len) gives the length of the message body (i.e., it does not include the message header).  The length field is an eight character string with the length in decimal and padded, as necessary, with leading zeros.  The valid message types are given in nasload.h.

Additional information can be found in the nasload source files nasload.cpp and nasload.h.

## NASLOAD

Nasload implements the NAS specific protocol.  Broadcom provides nasload, without warranty, to help our customers build their own download program.  Nasload is a simple command line program that runs on both Windows and Linux.

### Dependencies

Nasload calls out to two external programs.  For CFE clients, nasload calls out to a tftp client.  Both Microsoft Windows and Linux provide tftp clients.  For pivot boot image clients, nasload calls out to nasunpack which is provided by Broadcom.

### Command Line

Nasload has several command line arguments that can be used to customize its behavior.  Please see nasload’s usage text for more information.  When customized and packaged appropriately, nasload should run and download firmware when executed without any arguments (e.g.,filenames).  The name of the file to download can be compiled into nasload, and nasload will use broadcasts (on the local subnet) to find any ailing BroadNAS devices.  However, options allow the user to provide an alternate file name or to give a specific IP address or machine name.  By default, nasload runs interactively, but with the correct combination of functions, nasload can be executed by another program.  By default, nasload will run indefinitely and handle any download requests that it receives.  However, each request is handled serially.

The user should be careful to provide the correct type of file as input to nasload.  When communicating with the pivot boot client, nasload requires the input file to be a NAS file. Nasload uses the program nasunpack (which must be available in the current path) to parse out the disk portion of the split NAS file (broadnas_split.nas).  However when communicating with CFE clients, nasload will download any type of file without examining its contents.  In some circumstances, CFE will detect that an invalid type of file has been downloaded.

### Compiling

Nasload can be compiled for both Windows and Linux.  Broadcom provides a Microsoft Developer’s Studio workspace and project file for nasload.  Broadcom provides a Windows executable for nasunpack.  Broadcom also provides a makefile for building the Linux version, and a Linux executuable for nasunpack.

All the source files are located in nasoc/hostsrc/nasload.  Pre-compiled Linux executables are available under nasoc/tools, and pre-compiled Windows executables are available under nasoc/tools/win.