NASoC Software Release 2.0

Release Notes

- Contents

1.	Changes to RC1 from  Beta4	4

1.1	CFE Changes	4

1.2	NAS Software Changes	4

1.3	Known Limitations	4

2.	Changes to Beta4 from  Beta3	5

2.1	CFE Changes	5

2.2	NAS Software Changes	5

2.3	Known Limitations	5

3.	Changes to Beta3 from  Beta2	7

3.1	CFE Changes	7

3.2	NAS Software Changes	7

3.3	Known Limitations	8

4.	Changes to Beta2 from  Beta1	9

4.1	CFE Changes	9

4.2	NAS Software Changes	9

5.	Changes to Beta1 from  Alpha8	10

5.1	CFE Changes	10

5.2	NAS Software Changes	10

6.	Changes to Alpha8 from  Alpha7	12

6.1	CFE Changes	12

6.2	NAS Software Changes	12

7.	Changes to Alpha7from  Alpha6	13

7.1	CFE Changes	13

7.2	NAS Software Changes	13

8.	Changes to Alpha6 from  Alpha5	14

8.1	CFE Changes	14

8.2	NAS Software Changes	14

9.	Changes to Alpha5 from  Alpha4	15

9.1	CFE Changes	15

9.2	NAS Software Changes	15

10.	Changes to Alpha4 from  Alpha3	16

10.1	CFE Changes	16

10.2	NAS Software Changes	16

10.3	Notes	16

# Changes to RC1 from  Beta4

## CFE Changes

- Updated documentation nvram_list.txt. Includes information about

  - cfe_wait_led_gpio

  - cfe_wait_on_restore

- Disable VLAN in CFE. This fixes the bug, where there was no network connectivity from CFE after a soft reboot.

- Added support for simultaneous switching outputs (SSO) memory test to CFE

## NAS Software Changes

- In manufacturing rc, dynamic WAN protocol would not work correctly. This has been fixed now.

- Fixed a bug where users were not able to create a striped pool

- SATA drive showed up as USB on UI. This has been fixed now.

- Updates to Resource Guide to include information about 4785

- Link status for 4785 was not being updated on the web page correctly. This has been fixed now

- Ethernet hot plug support was broken in 4785. This has been fixed now

- Fixed a bug where SATA disks are allowed to be claimed as FAT 32

## Known Limitations

- Open SSL hardware acceleration for RC4 is not available.  Hardware acceleration is available for AES, DES, and 3DES.

- RAID 5 hardware acceleration support for XOR is not available.

- SATA SMART and hot-plug are not functional. The new libata driver inherently has no support for SMART and hot-plug and is being added.

- Cannot change ACLs for domain users in domain member mode.

- Not all SATA drives are accessible on a BCM4780 based system.

# Changes to Beta4 from  Beta3

## CFE Changes

- Added mechanism to overwrite IP address and netmask to anything other than lan_ipaddr and lan_netmask in mfg and config mode. CFE will set up the IP address during configuration initialization and manufacturing with the 	value from two new NVRAM variables.

  - cfe_mfg_ipaddr & cfe_mfg_netmask.

## NAS Software Changes

- Replaced SATA driver. Frodo driver has been replaced with libata.

- PWM fan driver enhancements:

  - Added fan_freq and fan_duty_cycle proc interface to improve the usability.  Look at resource guide for mode details.

  - The proc entries are in /proc/sys/dev/pwm directories.Fan entries are controlled by proc entries in /proc/sys/dev/fan.

- Added new NVRAM variables

  - nasocflags, temp_device, fan_device. Fan and temp modules will be loaded based on these NVRAM variables.

- On Beta2 and Beta3 pressing the soft power down button had no effect. This has been fixed now

- Query users fails when user is created without a description. This has been fixed now.  Only affects newly created users on Beta3. Delete any users created using Beta3 firmware.

- Previously UI allowed only English characters for pool name, disk name etc. Changed the validation checks to allow for multiple language support. Characters not allowed are  ; : / \ [ ] " < > * + = | ? ,

## Known Limitations

- Open SSL hardware acceleration for RC4 is not available.  Hardware acceleration is available for AES, DES, and 3DES.

- RAID 5 hardware acceleration support for XOR is not available.

- Disable SMART control for SATA because it causes data transfer problems. Set  nvram variable ‘disk_health_query_ctrl=never’ .

- SATA hot-plug is not functional.

- ACL does not work with Windows domain controller, but works fine with another NAS as PDC.

- Stress tests with multiple clients writing/reading to one NASoC can fail after
10+ hours.

- Some external PCI devices cause conflicts with integrated cores.  This has been seen with external USB controllers.  This issue is still under investigation.  As a temporary workaround, an external Gigabit NIC (5705 based) can be used.

# Changes to Beta3 from  Beta2

## CFE Changes

## NAS Software Changes

- MAC Address can be specified without colons. Ex: 1122334455667788 is treated as 11:22:33:44:55:66:77:88. This is to enable hexadecimal bar codes that does not contain colons

- Fan entries are controlled by proc entries in /proc/sys/dev/fan. They are

  - PWM1InputPeriodCount

  - PWMOutputPeriodCount

  - PWMOutputDutyCycleHighCount

- Temperature sensor entries are controlled by entries in  /proc/sys/dev/sensors/g751-i2c-0-48

  - temperature in Celsius

- If the NAS is rebooted while trying to encrypt a non-encrypted pool, then the UI will not let the user correctly mount the pool.  This does not occur when a pool is created from scratch as an encrypted pool.

- Event Notification: Fixed event notification. Some strings were not being put in /tmp/notification. Look at resource guide for more details.

- Event Notification: When a new disk arrives a signal will be sent to process and New Disk notification will be placed in /tmp/notification. Process name is obtained from nvram variable “event_process”. Look at Resource Guide for more details.

- Added support for USB testing. Four different modes of USB tests will be setup for EHCI based on NVRAM variables.  usb_test (1 to 4) sets up the test to be run and usb_test_port sets the port on which to run the test.  This will be done only if usb_test NVRAM variable is set and hence a console access is required to switch between test and non-test mode.

- Added methods for changing share_description. Overloaded web hook change_access for_cifs_share_optimized. See UI customization guide for more details

- Bug: In a rotating backup scenario, two USB disks (say USB1 and USB2) are added as mirrors to a IDE base. When USB1 is taken out and then USB2 is taken out and then USB1 is put back in, it corrupts information on the disk. This has been corrected.

## Known Limitations

- Open SSL hardware acceleration for RC4 is not available.  Hardware acceleration is available for AES, DES, and 3DES.

- RAID 5 replacement of disk is not fully functional.  No web GUI provided for creating RAID 5 and replacement of disk in RAID 5.  Hardware acceleration support for XOR not available.

- Disable SMART control for SATA because it causes data transfer problems. Set  nvram variable ‘disk_health_query_ctrl=never’ .

- SATA hot-plug is not functional.

- ACL does not work with Windows domain controller, but works fine with another NAS as PDC.

- Stress tests with multiple clients writing/reading to one NASoC can fail after
10+ hours.

- Some external PCI devices cause conflicts with integrated cores.  This has been seen with external USB controllers.  This issue is still under investigation.  As a temporary workaround, an external Gigabit NIC (5705 based) can be used.

- Integrated fan and temperature control based on sensord  (see Resource Guide for the sensor daemon) is not supported yet.

# Changes to Beta2 from  Beta1

## CFE Changes

## NAS Software Changes

- Increased Samba receive buffer size. This clears some TCP zero window situations. This reduces connection drop outs.

- Increased Samba send buffer size. Improves performance when doing simultaneous reads and writes.

- RAID 5 recovery process would not resume if power is lost and comes back up again. This is fixed now.

- On 32M platform copying 2G+ files to a Reiser FS USB disk was failing because of memory pressures. This has been fixed now.

- Fixed a problem in which NASOC as a domain member was not able to reconnect to PDC after a reboot.

- Enabled loading of ethernet modules in manufacturing image for 4785 based systems.

- Added NVAM variable ‘ide_spinup_timeout’ to control IDE spinup timeout.  The driver waits for 30 seconds by default for IDE drives to spin up if this NVRAM variable is not defined.  If defined, this NVRAM variable defines the time.

# Changes to Beta1 from  Alpha8

## CFE Changes

- Created separate NVRAM text files for BCM4785 Eval board for NAS and

NAS-Router.  Separate cfenas.bin images are created for BCM4780 and BCM4785

and are suffixed with _bcm4780 and _bcm4785 respectively.

- Rescue mode GPIO was fixed to be active-high.  Added support to select

polarity of rescue_gpio with rescue_gpio_polarity NVRAM variable.

- CFE version changed to 1.4.0.

## NAS Software Changes

- Added support to map hidden,archive and system file attributes from Windows to BroadNAS. Enabling and disabling the mapping is controlled by the following NVRAM variables.

  - smb_create_mask - Default permissions when a file is created in share. Defaults to Samba defaults to 0744

  - smb_map_archive  This controls whether the DOS archive attribute should be mapped to the UNIX owner execute bit. Default = Yes

  - smb_map_hidden - This controls whether DOS style hidden files should be mapped to the UNIX world execute bit. Default=No

  - smb_map_system This controls whether DOS style system files should be mapped to the UNIX group execute bit. Default=N

- Added support for Telnet daemon.  This is controlled by NVRAM variable, telenet_support and Telnet is enabled if this NVRAM variable is defined as "enabled".

- CFE upgraded from web GUI can be done such that the existing embedded NVRAM region is maintained. To this this, the CFE trx file needs to be created as

trx -f 0x0B -o cfenas.trx cfenas.bin

- Added capability of providing description for users added in UBSA mode.

- Added SMBus driver and RTC support for SMBus.

- Support for loading kernel modules for BCM4780 and BCM4785 based systems based on boardtype.  For BCM4785 based system, the boardtype NVRAM variable needs to be set at 0x4785.

- Added two nvram variables.

  - event_process: will have the name of the process which will receive SIGUSR1 whenever create user_note is called. Summary of the event will be available from /tmp/notification. Receiver of this event notification is advised to take an exclusive lock and read the contents. If not set, then no signal will be sent.

  - smb_notifcation: will have the name of the process which will receive SIGUSR1 whenever smb.conf is modified. If not sent then no signal will be sent.

- RAID 5 support is enabled.  Please see known issues.

- Extended GPIO driver to be built for BCM4785 with 32 GPIOs and support for controlling muxing via config proc entry.  Now "mux control" can be used along with other configuration settings for GPIO for BCM4785 based systems. BCM4780 based systems will continue to be as it was before.

- Added support for early setting of GPIO muxing for BCM4785 based system with cfe_gpio_mux_ctl and kernel_gpio_mux_ctl.  This sets the 32 bit value of the mux control register.

# Changes to Alpha8 from  Alpha7

## CFE Changes

## NAS Software Changes

- Same feature set as 1.3.1 RC1

- Support for Dynamic DNS

- Support for Bonjour – Detection of Macintoshes on the network.

- Added support for OpenSSL. Uses the hardware accelerator on 4785.

- Increased timeout from 50msec to 500msecs in the identify function for SATA controller to allow slower drives to respond within timeout.

# Changes to Alpha7from  Alpha6

## CFE Changes

- Gigabit Ethernet descriptor table fix. When writing to SATA driver through a remote client Gigabit ethernet interface has been observed to stop receiving packets.  This has been fixed now.

## NAS Software Changes

- 4785 PWM driver to monitor and control fan speed.  Interface to the driver is done via the following proc entries in /proc/sys/dev/fan/:

  - TempCtlClk

  - PWMOutputPeriodCount

  - PWMOutputDutyCycleHighCount

  - PWM1InputPeriodCountEnabled Smart for SATA

- Fix the "No such device or address" error when trying to access a SATA drive on port 3. Port 3 now accessible.

- Added support for 538x Robo switch driver in linux kernel.

- Changed the GigE driver in CFE and linux to user et0macaddr NVRAM variable for MAC address rather than using value from EEPROM.

# Changes to Alpha6 from  Alpha5

## CFE Changes

## NAS Software Changes

- Added share description to web hook for create share. Customers can now provide a share name as well as a description for the share.

- Enabled Smart for SATA

- CFE GigE driver working in RGMII mode.

- Reduce the FIFO burst size for ports 0-3 (SATA) and 15 (XOR)from 128 to 64 bytes.  Preliminary testing suggests that reducing the burst size resolves the following observed problems:

  - Kernel panics observed when copying a file from ATA to SATA using the console.

# Changes to Alpha5 from  Alpha4

## CFE Changes

## NAS Software Changes

- Added support for the 538x GbE switch.

- New NVRAM variable, "nasocflags", added.  Here are the values 

0x00001 /* This board has a GigE MAC or core */
0x00002 /* This is a NAS-Router board */
0x00004 /* This board has a GigE Robo switch or core */
0x00008 /* This board has wireless LAN support */
0x00010 /* This board has a SATA frodo or core */

# Changes to Alpha4 from  Alpha3

## CFE Changes

## NAS Software Changes

- CPU  up and running

- PCI enumeration done

- SATA R/W working

- USB EHCI and OHCI devices now recognized and operational

- IDE  controller working. Accessing IDE.

- GigE controller running at speed.

- Programming added for the 5461 PHY to enable energy detect, activity, and speed LEDs correctly on bringup board, as well as link ready signal to the GbE core.

- Added vanilla version of the GbE eeprom image to the Linux root file system image

- Default kernel modules loaded by the Linux startup code now includethe GbE, security, and streaming encryption drivers.

- PWM not tested.

## Notes

The BCM4785 firmware consists of the boot loader plus Linux kernel and application softwaer.  These must be programmed into the flash as follows:

cfenas.bin        offset 0x00000

broadnas.trx      offset 0x40000 (256K)

These can be combined into a single image using the flashpack utility.  To create an 8 MB flash image with name flash_8mb.img,  use the following command:

perl flashpack.pl \

--boot-image cfenas.bin \

--firmware-image broadnas.trx \

--firmware-offset 0x40000 \

--flash-fill 0xff \

--flash-image flash_8mb.img \

--flash-size 8