NASoC Software Release 2.0

Release Notes

- Contents

1.	Release Contents	1

2.	Release 2.0 Major Features	2

3.	Known Issues	3

# Release Contents

This release contains the following:

- NASoC reference platform software- BroadNAS

- Release notes

- User's Guide

- QuickStart Guide

- Manufacturing test document

- Developers Resource Guide

- UI Customization Guide

# Release 2.0 Major Features

- Support for BCM4785/81 added.  Same code base for BCM4785/81 and BCM4780.  The 8 MB firmware binary is the same for both.  CFE boot loader and split image binaries are build separate for BCM4780 and BCM485/81 based systems.

- Support for integrated SMBus controller provided as SMBus driver.  RTC driver uses SMBus in BCM4785/81 based systems.  RTC is not fully functional.

- Web GUI localization mechanism for the front-end of the web pages has been changed in 2.0.  The back-end localization uses the same mechanism as before.  With the old mechanism, multiple web pages were required for different languages.  This has been modified where only the text files that need to be localized are required to be provided and not the entire web page.  Details of this will be provided in the update UI customization guide.

- RAID 5 support enabled in the firmware.  This is not complete yet.  The XOR is still done via software.  The XOR driver for the hardware XOR engine integrated in the BCM4785/81 is available but not integrated with the RAID 5 software.  The routines to create RAID 5 and to replace a disk in RAID 5 are available in the code, but do not work as expected.  No web GUI is available for this either.

- Support for 32 GPIOs for BCM4785/81 based system.  Same driver works for both BCM4780 and BCM4785/81 and provides support for 8 and 32 GPIOs respectively.  Also, for BCM4785/81 support additional control for muxed GPIO pins is available in the proc entry.

- Dynamic DNS support.

- OpenSSL library with hardware acceleration support for AES, DES, 3DES.  RC4 hardware acceleration will be added later.  Available only for split or 16M Flash images.

- Share and user description can be added while creating shares or users.

- GigE Robo-Switch control device driver.  VLAN support not tested yet.

# Known Issues

- RAID 5 is functioning properly and there is no front-end GUI.  The XOR acceleration is also not available for RAID 5.

- Driver for temperature sensor G751 is not available.